ECE799 Final Project Report
Finding the Parasitic Inductances in a VLSI System Utilizing an Efficient Tree-Code Algorithm

Jeremy Tanumihardjo

Fall 2008

Project Advisor: Nick Hitchon

Academic Advisor: Kewal Saluja

ABSTRACT

Area and power have become larger motivators recently than in the past in both the general purpose and ASIC environment. With the advent of sub-45 micron technologies, more transistors can be fit into a specified area, creating much more dense and complex circuitry. There has also been an increased pressure in time-to-market deadlines causing shorter product cycles.  These motivators have pushed the necessity for better VLSI-level algorithms that can find the trade-offs involved with minimizing the total area of the circuit while keeping delay constrained.

Most VLSI algorithms, such as those used in Cadence, focus only on finding the parasitic capacitances within a circuit, which up until recently have been the dominating forces that cause delays due to stray electric fields.  Now that circuits have become much denser and more complex, the magnetic field effects are starting to become significant as well, causing these so-called parasitic inductances.  These effects become more profound as technologies get smaller that more accurate algorithms become necessary, yet the speed of them must remain the same.  Typically these algorithms are very fast compared to other parts of the design flow, but the accuracy is becoming much worse.  We desire an algorithm that can calculate the parasitic inductances within a circuit efficiently, necessitating both accuracy and speed

We use an existing treecode algorithm from previous works provided by Professor Andrew J. Christlieb.  We consider this an efficient algorithm in that it ensures the necessary accuracy we desire at reasonable computation speed.

Motivation

Given an applied voltage, the parasitic inductance limits the rate at which the current can change.  Given an applied current, the parasitic capacitance limits the rate at which the voltage can change.  Both of these effects combined are the parasitic components that form resonant circuits that lead to ringing and EMI.

The original tree code was used to calculate the electric field effects given a simulated plasma breakdown event.  The code is written in C++.  Translations from the electric field equations to the magnetic field equations are needed for the project.  This includes necessary changes to constants as well.

Implementation

Much of the project timeline (one semester) was spent familiarizing with the treecode and being able to run the simulations.  The program outputs many field strengths at many locations on the grid.  It is assumed to be a 2D environment.  The outputs were parsed using MATLAB to generate reasonable graphs.  The program is 23 pages and small adjustments to code can cause results to go wrong.  As such, much care was put into modifying the equations accordingly.

Project’s Open End

There was not enough time to modify all of the equations necessary to provide for reasonable magnetic inductance calculations.  Further research is necessary both into the code and the algorithms in the program.  In particular, much of the original code’s interpretation of the simulation assumes electric monopoles.  Whereas, in the magnetic translation:  There is no such thing as a magnetic monopole.  Therefore models for the magnetic dipole, as well as how they can form into other m-pole structures are necessary for the code to be run efficiently.

The code’s efficiency is mostly comes from being able to combine several closely located particles into another modeled equation.  Please read through the reference material to understand these equations.

Left for Future Work

For those that wish to pick up where this has left off.  It will be necessary to quickly familiarize with the original tree code and be able to run it and understand the results that come from it.  After this, one needs to look into the proper electric field to magnetic field translation equations, particularly those that can use combined models of tightly packed particles.  Once the necessary modifications have been made, simulations can be done giving the relevant data.

References

[1] A Particle Method and Adaptive Treecode for Vortex Sheet Motion in Three-Dimensional Flow.  Lindsay, Keith. Krasny. June 19, 2001.

[2] A treecode algorithm for simulating for simulating electron dynamics in Penning-Malmberg trap.  Christlieb, A.J.  Krasny, R.  Verboncoeur, J.P. July 27, 2004.

[3] Efficient Particle Simulation of a Virtual Cathode Using a Grid-Free Treecode Poisson Solver.  Christlieb, Andrew J.d  Krasny, Robert.  Verboncoeur, John P.  April 2004.

[4] Grid-Free Plasma Simulation Techniques.  Christlieb, Andrew J.  Krasny, Robert.  Verboncoeur, John P.  Boyd, Iain D.  April 2006.

For all following code, the majority of it was written by Professor Christlieb’s group, modifications done for the project specifically are commented as necessary

Appendix A: Tree Code Header File

/*---------------------------------------------------------------------------*/

//  treecode_2D.h                                                            //

//  Code by Jerry Emhoff                                                     //

/*---------------------------------------------------------------------------*/

#ifndef treecode_2D_h

#define treecode_2D_h

#include <fstream>             //  File stream library (std. C)

#include <iostream>            //  File stream library (C++)

#include <iomanip>

#include <cmath>              //  Math library

#include <ctime>

#include <cstring>

#include "constants.h"

#include "RandL.h"

using namespace std;

// Structure for panel quantities

struct PANEL{

double x0;      // Starting x-point

double x1;      // Ending x-point

double y0;      // Starting y-point

double y1;      // Ending y-point

double str;     // Panel strength

double xnrm;    // X-normal

double ynrm;    // Y-normal

double refpot;  // Reference potential or slope

double midx;    // x midpoint

double midy;    // y midpoint

int type;       // Type of panel - Dirichlet or Neumann

double* points; // Integration points

double refx;    // Reference x-midpoint (for offset panels)

double refy;    // Reference y-midpoint (for offset panels)

};

struct BOUNDARY{

int type;

double length;

double x0;

double y0;

double x_normal;

double y_normal;

double refpot;

int direction;

};

// Structure for particle quantities

struct PARTICLE{

double charge;  // Charge on the species (i.e. +1, -1, +2, etc.)

double x;       // x-position of the particle

double y;       // y-position

double u;       // Velocity in the x-direction

double v;       // Velocity in the y-direction

double xforce;  // Field in the x-direction

double yforce;  // Field in the y-direction

// Particle weight - i.e. the amount of real particles represented

double weight;

double tot_charge;  // Charge x Weight

double alpha;     // Amount of energy that goes into ionization

double mu;        // Mobility of the particle

bool movable;		// Is the particle an ion (stationary) or electron (movable)

int boxcol;		// The box column that the particle is in for recombination. Required for ions only.

int boxrow;		// The box row that the particle is in for recombination. Required for ions only.

};

// Structure for tree properties

struct TREE {

int nummemb;      // Number of member particles

double x0;        // Left x-limit

double x1;        // Right x-limit

double y0;        // Bottom y-limit

double y1;        // Top y-limit

double xmid;      // x midpoint

double ymid;      // y midpoint

double aspect;    // Aspect ratio (width/height)

double sqradius;  // Square of the box radius

int children[NUMCHILD]; // Child trees of the current tree

int *members;     // Index of member particles

double* moments;  // Moments of the cluster

};

///////////////////////////////////////////////////////////////////////////////

// Function declarations

int box_calc_tree();  // Main treecode program

int box_calc_ds();    // Main Direct summation program

// Box domain boundary setup

int setboundaries(int toppan, int botpan, int sidepan, double dlength,

double dheight, PANEL* pan, double maxpot, int usegq);

// Initializes particles in the box domain

void boxparticles(int npart, double dlength, double dheight,

PARTICLE* part, double density);

//////////////////////////////////////////////

// Dan's new functions

// Mobility calculations

double calcalpha(double totefield);

double calcmu(double totefield);

// Particle Management

void addparticle(int & npart, PARTICLE* & part, double charge, double x, double y, double weight, bool movable, double dlength, double dheight);

void removeparticle(int & npart, PARTICLE* & part, int partnum, int* & removedarray, int & nremoved);

void splitparticle(int & npart, PARTICLE* & part, int partnum, double delta);

void addion(int & npart, PARTICLE* & part, double charge, double x, double y, double weight, bool movable, double dlength, double dheight, int* & ion, int & nion);

// Ion Recombination

int getrow(double y, double dheight);

int getcol(double x, double dlength);

////////////////////////////

// Panel functions

// Panel initialization function

void panelinit(PANEL* panl, double x0, double x1, double y0, double y1,

double refpot, double xnrm, double ynrm, int type, double* gpts,

int usegq);

// Fills the panel effects array using exact integration

int panelconstantsreal(int numpan, PANEL* pan, double* panelarray,

int invert);

// Fills the panel effects array using GQ integration

int panelconstantsgauss(int numpan, PANEL* pan, double* panelarray,

int invert);

// Solves for the panel strengths using direct summation

int panelsolvereal(int numpan, int npart, double* panelarray,

PARTICLE* part, PANEL* pan, int invert);

// Solve for the panel strengths using the treecode.

int panelsolvetree(int numpan, int npart, int nterms, double acc,

double* panelarray, PARTICLE* part, PANEL* pan,

TREE* tree, int invert);

////////////////////////////

// Output functions

// Output particle properties

void output_particles(int npart, PARTICLE* part, string filename);

// Plots the potential field using either DS or TC

int plotpot(int nxnodes, int numpan, int npart, int type, int nterms, double acc,

double dlength, double dheight, PANEL* pan, PARTICLE* part,

TREE* tree, int usegq);

// Plot the electric field using the treecode

void plot_efield_tree(int nxnodes, int npart, int numpan, int nterms, int maxmemb,

double acc, double dlength, PARTICLE* part, PANEL* pan,

TREE* tree, int usegq);

void plot_efield_ds(int nxnodes, double dlength, int npart, int numpan,

PARTICLE* part, PANEL* pan, int usegq);

////////////////////////////

// Potential calculation

// Calculates the potential at a point using direct sum

double calcpot(double x, double y, int numpan, int npart, PANEL* pan,

PARTICLE* part, int usegq);

// Calculates the total potential at a point using the treecode

double calctreepot(double x, double y, int numpan, int npart, int nterms,

double acc, PANEL* pan, PARTICLE* part, TREE* tree,

int usegq);

// Calculate the potential from the panels

double potential_panel(double x, double y, int numpan, PANEL* pan, int usegq);

////////////////////////////

// Force / electric field calculation

// Electric field / force from the panels

int efield_panels(double x, double y, double* px_force, double* py_force,

int numpan, PANEL* pan, int usegq);

// Determine if a point is near the boundary

int nearboundary(double x, double y, double offset, int* px_bound, int* py_bound);

// Calculates the force on a point from the panels

void force_panel(double x, double y, double* px_force, double* py_force,

int numpan, PANEL* pan);

// Computes the force on a point from all panels using GQ integration

void force_panel_GQ(double xp, double yp, double* px_force, double* py_force,

int numpan, PANEL* pan);

// Computes the force on a particle due to other particles using direct sum

void forceparticlereal(int ind, int npart, double* xfor, double* yfor,

PARTICLE* part);

void forceparticlereal_point(double x, double y, int npart, double* xfor,

double* yfor, PARTICLE* part);

// Computes the force on a particle from all panels using exact integration

void forcepanelreal(int numpan, PARTICLE* ppart, PANEL* pan);

////////////////////////////

// Treecode exclusive functions

// Creates the tree

int treemake(int num, int maxmemb, double dlength, double dheight,

PARTICLE* part, TREE* tree);

// Deletes the tree particle arrays

void treeempty(int numtree, TREE* tree);a

// Calculates the moments of all tree clusters

void treemoments(int numtree, int nterms, TREE* tree, PARTICLE* part);

// Computes the force on a particle or point using the treecode

double treeforce(int ind, int p, int nterms, double x, double y, double acc,

double* xfor, double* yfor, TREE* tree, PARTICLE* part);

// Computes the potential at a point from particles using the treecode

double treepot(int ind, int nterms, double x, double y, double acc,

TREE* tree, PARTICLE* part);

// Plots the tree clusters

void treegraph(int level, int numtree, TREE* tree);

// Initialize the tree

void treeinit(TREE* tree1, double x0, double x1, double y0, double y1);

// Divide a cluster into two pieces

int treedivide(int dir, TREE* split, TREE* c1, TREE* c2, PARTICLE* part);

// Shrink a cluster around its members

void treeshrink(TREE* tree1, PARTICLE* part);

// Compute the force using a Taylor expansion

void treetaylorforce(int nterms, double dx, double dy, double rs,

double* xfor, double* yfor, double* moment);

// Compute the potential using a Taylor expansion

double treetaylorpot(int nterms, double dx, double dy, double rs,

double* moment);

/*****************************************************************************/

// Function evaluations

double arctan(double x1, double x0, double y1, double y0, double normal);

double arctan2(double x1, double x0, double x,

double y1, double y0, double normal);

double logint(double x1, double x0, double y1, double y0);

double invval(double x1, double x0, double y1, double y0);

double lnrs(double x1, double x0, double y1, double y0);

void gaussvals(int num, double points[], double weights[]);

void invertmatrix(double* a, int n);

int printmat(double* parr, int size, int prec);

int gmres(int m, double* a, double* b, double* x);

int inttostr(int a, char *str);

int gmresclean(int num, double *a, double *b, double *x);

int plotarray(int nx, int ny, string name, double* array);

// Global variables

double del;       // Smoothing parameter

// Gaussian Quadrature variables

int npoints;      // Number of numerical integration points to use

int boundary_points;

double* wts;

BOUNDARY* boundaries; // Array of boundary vertices and sides

#endif //!defined(treecode_2D_h)

Appendix B: Tree Code Program

/*---------------------------------------------------------------------------*/

//  paneltest.cpp                                                            //

//  Code by Jerry Emhoff                                                     //

//  University of Michigan Department of Aerospace Engineering               //

//  Nonequilibrium Gas and Plasma Dynamics Group                             //

/*---------------------------------------------------------------------------*/

#include "treecode_2D.h"

int main()  {

// Initialize the random number generator

// Replace time(0) with an odd constant to obtain the same progression of

// numbers every time (for debugging)

//SetLCGSeed(static_cast<unsigned int>(time(0)));

SetLCGSeed(static_cast<unsigned int>(1));

box_calc_tree();

//box_calc_ds();

return(0);

}

int box_calc_tree()  {

/*****************************************************************************/

//  box_calc_tree - Create particles and calculate the forces on them using

//                  the treecode.

/*****************************************************************************/

int i;              // Index variable

int side = 0;       // Number of panels per side

int numpan = 0;     // Total number of panels.  Make this divisible by four.

int npart = 0;      // Number of particles in the simulation

int numtree = 0;    // Number of trees

int nterms = 4;     // Number of terms in the treecode Taylor expansion

int maxmemb = 8;    // Maximum number of particles per bottom level cluster

int maxtrees;       // Maximum number of clusters

int nxnodes = 129;  // Set the grid size for sampling the potential

int usegq = 0;      // Set the method for panel calculation

int invert = 1;     // Set whether or not to invert the panel effects matrix

// If zero, then GMRES is used to solve for

// panel strengths

// Time loop variables

int t;	             // Time step index value

double tstep = 1.0e-10; // Amount of time each time step travels.

//int numtstep = 1018;    // Number of time steps to perform

int numtstep = 1000;    // Number of time steps to perform

int logdata = 1;		  // Set to zero to prevent data being sent to the logfile every timestep.

// Debug variable for tau_d

double maxmu = 0.0;

// Charge movement variables

double totefield = 0.0; // Magnitude of the E field

double xdistmoved = 0.0; // keep track of how far the particle moved in the x direction

double ydistmoved = 0.0; // keep track of how far the particle moved in the y direction

double newweight = 0.0;  // new weight of a particle

// Particle split variables

double delta = 5.0e-5; // Smallest spatial distance to resolve.

int tempnumpart = 0;

double dlength;   // Domain length

double dheight;   // Domain height

double xfor;      // Temporary x-force

double yfor;      // Temporary y-force

double acc = 0.8; // Treecode acceptance parameter squared.

double clocktime; // Time computed using clock cycles

double dx;

double density; // Particle number density

time_t beg;       // Beginning time (in seconds)

time_t end;       // Ending time (in seconds)

clock_t begtime;  // Beginning time (in clock cycles)

clock_t endtime;  // Ending time (in clock cycles)

//////////////////////////////////////////////////

// My additional time variables

clock_t PanBoundSetupBegTime;

clock_t PanBoundSetupEndTime;

clock_t MaxtreesCalcBegTime;

clock_t MaxtreesCalcEndTime;

clock_t SolveTreeBegTime;

clock_t SolveTreeEndTime;

clock_t FindEFieldBegTime;

clock_t FindEFieldEndTime;

clock_t MoveSplitAddPartBegTime;

clock_t MoveSplitAddPartEndTime;

clock_t OutputDataBegTime;

clock_t OutputDataEndTime;

double* panelarray;     // Panel effects array

TREE* tree = NULL;      // Tree array

PANEL* pan = NULL;      // Panel array

PARTICLE* part = NULL;  // Particle array

// Initialize variables

del=1e-6;

usegq = 0;    // Indicate whether or not to use GQ panel integration

npoints = 8;  // Number of GQ points on each panel

// Domain and panel setup

dlength = 0.01;      // Set the domain length

dheight = 0.0001;    // Set the domain height

numpan = 256;       // Set the number of panels

side = numpan / 4;  // Set the number of panels on each side.

npart = 1;      // Set the number of particles

nxnodes = 129;      // Set the number of nodes for potential field sampling

density = 1.0e14	;   // Set the particle number density

// Note: All particles (ions and electrons) are contained in the part array.

//       Ions are again stored in the ion array to speed ion recombination calculations.

int nion = 0; // Initial number of ions

int* ion = NULL; // Ion array

ion = new int[nion];

// Read TC input file

ifstream in("tc-input.txt");

in >> nxnodes;

in >> nterms >> maxmemb >> acc >> del;

in >> numpan;

in.close();

side = numpan / 4;

dx = dlength / static_cast<double>(nxnodes);

/////////////////////////////////////////////

// Array allocation

part = new PARTICLE[npart];             // Allocate the particle array

pan = new PANEL[numpan];                // Allocate the panel array

panelarray = new double[numpan*numpan]; // Allocate the panel effect array

//maxtrees = static_cast<int>(3*npart/maxmemb+10); // Original maxtrees calculation, needs to be inside main loop

/////////////////////////////////////////////

// Domain setup functions

// Set up the boundaries and panels

cout << "Setting up boundaries" << endl;

setboundaries(side, side, side, dlength, dheight, pan, 0.0, usegq);

// Initialize the particles in the domain

cout << "Initializing particles" << endl;

boxparticles(npart, dlength, dheight, part, density);

//  time(&beg);       // Get the initial times

//  begtime=clock();

ofstream log("log.txt");

log << "nterms = " << nterms << endl;

log << "maxmemb = " << maxmemb << endl;

log << "acc = " << acc << endl;

log << "del = " << del << endl;

log << "numpan = " << numpan << endl;

// Setup Output Files

//ofstream dansoutputx("DansOutputx.txt");

//ofstream dansoutputy("DansOutputy.txt");

//ofstream dansoutputtype("DansOutputType.txt");

ofstream danslog("DansLog.txt");

ofstream dansoutputfinalx("DansOutputFinalx.txt");

ofstream dansoutputfinaly("DansOutputFinaly.txt");

ofstream dansoutputfinaltype("DansOutputFinalType.txt");

ofstream dansoutputfinalweight("DansOutputFinalWeight.txt");

/* // no longer really interested in this information

ofstream dansoutputpartstrx("DansOutputPartStrx.txt");

ofstream dansoutputpartstry("DansOutputPartStry.txt");

ofstream dansoutputboundstrx("DansOutputBoundStrx.txt");

ofstream dansoutputboundstry("DansOutputBoundStry.txt");

*/

danslog << "tstep: " << tstep << endl;

danslog << "delta: " << delta << endl;

danslog << "density: " << density << endl;

//////////////////////////////////////////////

// Start of main loop

for (t=0; t < numtstep; t++) {

time(&beg);       // Get the initial times

begtime=clock();

// My maxtrees calculation

/////////////////////////////////////////////////////////////////////////////////

MaxtreesCalcBegTime=clock();

int nelectrons = 0;

double avgeweight=0;

for (i=0; i<npart; i++) {

if (part[i].movable) {

nelectrons++; // I mix particles types in the part array. Electrons can be differentiated from ions by the movable property.

avgeweight+=part[i].weight;

}

}

avgeweight=avgeweight/nelectrons;

// Each time an electron is moved, an ion is placed behind it.

// Make room for each existing particle plus each electron to split

maxtrees = static_cast<int>(nelectrons+npart);

MaxtreesCalcEndTime=clock();

/////////////////////////////////////////////////////////////////////////////////

//cout << "t: " << t << " npart: " << npart << " nelectrons: " << nelectrons << " maxtrees: " << maxtrees << " tau_d: " << perm/(nelectrons*avgeweight*maxmu*elec) << " avg weight: " << avgeweight << endl;

cout << "t: " << t-1 << " npart: " << npart << " nelectrons: " << nelectrons << " avgeweight: " << avgeweight;

if (logdata) {

danslog << "t: " << t-1 << " npart: " << npart << " nelectrons: " << nelectrons << " avgweight: " << avgeweight;

}

PanBoundSetupBegTime=clock();

if (boundaries != NULL)

delete [] boundaries;

setboundaries(side, side, side, dlength, dheight, pan, 0.0, usegq);

panelconstantsreal(numpan, pan, panelarray, invert);

PanBoundSetupEndTime=clock();

SolveTreeBegTime=clock();

// Create the tree

tree = new TREE[maxtrees];

// Initialize all tree pointers to NULL

for (i = 0; i < maxtrees; i++)  {

tree[i].members = NULL;

tree[i].moments = NULL;

}

/////////////////////////////////////////////

// Original Main loop

numtree=treemake(npart, maxmemb, dlength, dheight, part, tree);

treemoments(numtree, nterms, tree, part);

// Solve for the panel strengths

panelsolvetree(numpan, npart, nterms, acc, panelarray, part,

pan, tree, invert);

SolveTreeEndTime=clock();

FindEFieldBegTime=clock();

// Find the E fields acting on the particles

for (i = 0; i < npart; i++)  {

// Only need to worry about finding the force on the electrons, since the ions are not movable.

if (part[i].movable) {

// Field due to the panels

efield_panels(part[i].x, part[i].y, &part[i].xforce,

&part[i].yforce, numpan, pan, usegq);

// Use the treecode to compute the force

xfor=0.0;

yfor=0.0;

treeforce(0, i, nterms, part[i].x, part[i].y, acc, &xfor,

&yfor, tree, part);

/* // no longer really interested in this information

if (t==numtstep-1) { // Output the field seen from other particles at a specifiedr timestep

dansoutputpartstrx << setiosflags(ios::fixed) << setprecision(15) << xfor*charge_constant << endl;

dansoutputpartstry << setiosflags(ios::fixed) << setprecision(15) << yfor*charge_constant << endl;

dansoutputboundstrx << setiosflags(ios::fixed) << setprecision(15) << part[i].xforce << endl;

dansoutputboundstry << setiosflags(ios::fixed) << setprecision(15) << part[i].yforce << endl;

}

*/

// The original code calculated the force on the particle by multiplying the field by the charge

// This is no longer necessary since the movement code uses the field itself rather than the force.

// Therefore, part[i].xforce and part[i].yforce actually refer to the electric field in my version

// of the code.

part[i].xforce+=xfor*charge_constant;

part[i].yforce+=yfor*charge_constant;

/*

// Multiply by the particle weight

part[i].xforce*=part[i].tot_charge;

part[i].yforce*=part[i].tot_charge;

*/

//part[i].xforce+=xfor;

//part[i].yforce+=yfor;

// part[i].xforce should now hold the x field acting on the particle and

// part[i].yforce should now hold the y field acting on the particle

totefield = sqrt(part[i].xforce*part[i].xforce + part[i].yforce*part[i].yforce);

part[i].alpha = calcalpha(totefield);

part[i].mu = calcmu(totefield);

}

}

FindEFieldEndTime=clock();

/* // No longer really interested in this information

if (t==numtstep-1) { // Output the field seen from other particles at a specifiedr timestep

dansoutputpartstrx.close();

dansoutputpartstry.close();

dansoutputboundstrx.close();

dansoutputboundstry.close();

}

*/

// By this point, all the fields and mobilities should have been calculated

// Move the particles using Euler integration

MoveSplitAddPartBegTime=clock();

tempnumpart = npart;

for (i = 0; i < tempnumpart; i++)  {

if (part[i].movable) {

xdistmoved = part[i].charge*part[i].mu*part[i].xforce*tstep;

ydistmoved = part[i].charge*part[i].mu*part[i].yforce*tstep;

// Increase particle weight based on alpha

totefield = sqrt(part[i].xforce*part[i].xforce + part[i].yforce*part[i].yforce);

// Old newweight formula. I don't think we are supposed to be multiplying by the particle weight in the exponential

//newweight = part[i].weight*exp(part[i].alpha*part[i].weight*elec*inveiz*totefield*sqrt(xdistmoved*xdistmoved + ydistmoved*ydistmoved));

newweight = part[i].weight*exp(part[i].alpha*elec*inveiz*totefield*sqrt(xdistmoved*xdistmoved + ydistmoved*ydistmoved));

// Place ions down behind the particle.

// If statement should fix bug where if the weight is equal to zero due to lack of precision,

// will not add a particle with zero weight.

if (newweight-part[i].weight > 0.0) {

addion(npart, part, 1.0, part[i].x+xdistmoved/2.0, part[i].y+ydistmoved/2.0, newweight-part[i].weight, false, dlength, dheight, ion, nion);

}

// For now, if particle would hit the right or left boundaries, remove it from the simulation.

// Make upper and lower boundaries periodic

//

// Currently (9/11/07), if particles hit the boundaries, stop the simulation rather than removing the particles

if (part[i].x + xdistmoved < 0) {

//removeparticle(npart, part, i, removedarray, nremoved);

cout << "Particle hit left boundary!" << endl;

danslog << "Particle hit left boundary!" << endl;

return(0); // For now, just stop the simulation rather than making periodic

} else if (part[i].x + xdistmoved > dlength) {

//removeparticle(npart, part, i, removedarray, nremoved);

cout << "Particle hit right boundary!" << endl;

danslog << "Particle hit right boundary!" << endl;

return(0); // For now, just stop the simulation

} else {

part[i].x += xdistmoved;

if (part[i].y + ydistmoved < 0) {

part[i].y = dheight - ydistmoved + part[i].y;

cout << "Particle hit bottom boundary! " << part[i].y << endl;

danslog << "Particle hit bottom boundary! " << part[i].y << endl;

return(0); // For now, just stop the simulation rather than making periodic

} else if (part[i].y + ydistmoved > dheight) {

part[i].y = ydistmoved - dheight + part[i].y ;

cout << "Particle hit top boundary! " << part[i].y << endl;

danslog << "Particle hit top boundary! " << part[i].y << endl;

return(0); // For now, just stop the simulation rather than making periodic

} else {

part[i].y += ydistmoved;

}

double tempweight = 0;

tempweight = part[i].weight;

part[i].weight = newweight;

part[i].tot_charge = part[i].charge*part[i].weight*elec;

// Particle split code

if (newweight*elec > totefield*2.0*pi*perm*delta) {

//if (newweight*elec > 2*totefield*2*pi*perm*delta) {

//	cout << "Error: Particle weight more than doubled in one timestep!" << endl;

//}

splitparticle(npart, part, i, delta);

}

if (newweight > 2.0*tempweight) {

cout << "Particle weight more than doubled in one timestep!" << endl;

danslog << "Particle weight more than doubled in one timestep!" << endl;

return(0); // For now, just stop the simulation

}

}

}

//dansoutputx << part[i].x << ", ";

//dansoutputy << part[i].y << ", ";

//if (part[i].movable) {

//	dansoutputtype << "5" << ", "; // Idea for the future: Make size proportional to weight?

//} else {

//	dansoutputtype << "1" << ", ";

//}

// Just output the last step, simulations are getting too big to make movies in Matlab

if (t==numtstep-1) {

dansoutputfinalx << setiosflags(ios::fixed) << setprecision(15) << part[i].x << ", ";

dansoutputfinaly << setiosflags(ios::fixed) << setprecision(15) << part[i].y << ", ";

dansoutputfinalweight << setiosflags(ios::fixed) << setprecision(15) << part[i].weight << ", ";

if (part[i].movable) {

dansoutputfinaltype << "5" << ", "; // Idea for the future: Make size proportional to weight?

} else {

dansoutputfinaltype << "1" << ", ";

}

}

}

MoveSplitAddPartEndTime=clock();

OutputDataBegTime=clock();

if (t==numtstep-1) {

// Plot the magnitude of the electric fields

double k = 0;

double l = 0;

double fielddelta = 1e-8;

ofstream dansoutputfieldstr("DansOutputFieldStr.txt");

ofstream dansoutputfieldstrx("DansOutputFieldStrx.txt");

ofstream dansoutputfieldstry("DansOutputFieldStry.txt");

ofstream dansoutputfieldx("DansOutputFieldx.txt");

ofstream dansoutputfieldy("DansOutputFieldy.txt");

/*

for (k = 1; k < dlength/fielddelta; k++) {

for (l = 1; l < dheight/fielddelta; l++) {

double xfield = 0;

double yfield = 0;

efield_panels(k*fielddelta, l*fielddelta, &xfield,

&yfield, numpan, pan, usegq);

// Use the treecode to compute the force

xfor=0.0;

yfor=0.0;

treeforce(0, -1, nterms, k*fielddelta, l*fielddelta, acc, &xfor,

&yfor, tree, part);

xfield+=xfor*charge_constant;

yfield+=yfor*charge_constant;

totefield = sqrt(xfield*xfield + yfield*yfield);

dansoutputfieldx << k*fielddelta << ", ";

dansoutputfieldy << l*fielddelta << ", ";

dansoutputfieldstr << totefield << ", ";

dansoutputfieldstrx << xfield << ", ";

dansoutputfieldstry << yfield << ", ";

cout << "k: " << k << " l: " << l << endl;

}

}

*/

//for (k = 600000; k < 800001; k++) {

for (k = 0; k < 1000001; k++) {

l=5000;

double xfield = 0;

double yfield = 0;

efield_panels(k*fielddelta, l*fielddelta, &xfield,

&yfield, numpan, pan, usegq);

// Use the treecode to compute the force

xfor=0.0;

yfor=0.0;

treeforce(0, -1, nterms, k*fielddelta, l*fielddelta, acc, &xfor,

&yfor, tree, part);

xfield+=xfor*charge_constant;

yfield+=yfor*charge_constant;

totefield = sqrt(xfield*xfield + yfield*yfield);

dansoutputfieldx << setiosflags(ios::fixed) << setprecision(15) << k*fielddelta << ", ";

dansoutputfieldy << setiosflags(ios::fixed) << setprecision(15) << l*fielddelta << ", ";

dansoutputfieldstr << setiosflags(ios::fixed) << setprecision(15) << totefield << ", ";

dansoutputfieldstrx << setiosflags(ios::fixed) << setprecision(15) << xfield << ", ";

dansoutputfieldstry << setiosflags(ios::fixed) << setprecision(15) << yfield << ", ";

cout << "k: " << k << " l: " << l << endl;

}

dansoutputfieldstr << endl;

dansoutputfieldstrx << endl;

dansoutputfieldstry << endl;

dansoutputfieldx << endl;

dansoutputfieldy << endl;

dansoutputfieldstr.close();

dansoutputfieldx.close();

dansoutputfieldy.close();

dansoutputfieldstrx.close();

dansoutputfieldstry.close();

}

OutputDataEndTime=clock();

treeempty(numtree, tree); // Delete the tree particle arrays

if (tree != NULL) {

delete [] tree;

tree=NULL;

}

// Fill in the maximum number of columns as determined by the

// number of particles times the number of timesteps (maximum number of particles in the simulation)

// with NaN values so Matlab intreprets it correctly. There is probably a better way of doing this, but

// I don't know how to make Matlab output work with the added ions otherwise.

/*

if (t==0) {

for (i=npart;i<50000;i++) {

dansoutputx << "NaN, ";

dansoutputy << "NaN, ";

dansoutputtype << "NaN, ";

}

}

dansoutputx << endl;

dansoutputy << endl;

dansoutputtype << endl;

*/

if (t==numtstep-1) {

dansoutputfinalx << endl;

dansoutputfinaly << endl;

dansoutputfinaltype << endl;

dansoutputfinalweight << endl;

}

endtime=clock();    // Get the ending times

time(&end);

clocktime=(endtime-begtime)/static_cast<double>(CLOCKS_PER_SEC);

cout << " Time: " << clocktime << endl;

if (logdata) {

danslog << " TotalTime: " << clocktime;

danslog << " PanelandBoundarySetup: " << (PanBoundSetupEndTime-PanBoundSetupBegTime)/static_cast<double>(CLOCKS_PER_SEC);

danslog << " MaxtreesCalc: " << (MaxtreesCalcEndTime-MaxtreesCalcBegTime)/static_cast<double>(CLOCKS_PER_SEC);

danslog << " SolveTree: " << (SolveTreeEndTime-SolveTreeBegTime)/static_cast<double>(CLOCKS_PER_SEC);

danslog << " FindEField: " << (FindEFieldEndTime-FindEFieldBegTime)/static_cast<double>(CLOCKS_PER_SEC);

danslog << " MoveSplitAddPart: " << (MoveSplitAddPartEndTime-MoveSplitAddPartBegTime)/static_cast<double>(CLOCKS_PER_SEC);

danslog << " OutputData: " << (OutputDataEndTime-OutputDataBegTime)/static_cast<double>(CLOCKS_PER_SEC);

danslog << endl;

}

}

/*

dansoutputx.close();

dansoutputy.close();

dansoutputtype.close();

*/

danslog.close();

dansoutputfinalx.close();

dansoutputfinaly.close();

dansoutputfinaltype.close();

dansoutputfinalweight.close();

/////////////////////////////////////////////

// Calculate the total time in seconds

// For very long cases, this will always be correct.

/*

endtime=clock();    // Get the ending times

time(&end);

clocktime=(endtime-begtime)/static_cast<double>(CLOCKS_PER_SEC);

cout << clocktime << "\t\t" << static_cast<int>(end-beg) << endl;

log << clocktime << "\t\t" << static_cast<int>(end-beg) << endl;

log.close();

*/

/* Don't really care about this stuff at the moment.

// Output to files

cout << "Outputting to files" << endl;

output_particles(npart, part, "tree-partforces.txt");

// Rebuild the tree and panel info for data output

numtree=treemake(npart, maxmemb, dlength, dheight, part, tree);

treemoments(numtree, nterms, tree, part);

panelsolvetree(numpan, npart, nterms, acc, panelarray, part, pan, tree,

invert);

plotpot(nxnodes, numpan, npart, 1, nterms, acc, dlength, dheight, pan,

part, tree, usegq);

plot_efield_tree(nxnodes, npart, numpan, nterms, maxmemb, acc,

dlength, part, pan, tree, usegq);

*/

delete [] pan;        // Delete the panel array

delete [] panelarray; // Delete the panel effects array

delete [] part;       // Delete the particle array

delete [] boundaries; // Delete the boundary array

treeempty(numtree, tree);

if (tree != NULL)

delete [] tree;

return(0);

}

//int box_calc_ds()  {

/*****************************************************************************/

//  box_calc_ds - Create particles and calculate the forces on them using

//                direct summation.

/*****************************************************************************/

/*

int i;              // Index variable

int side = 0;       // Number of panels per side

int numpan = 0;     // Total number of panels.  Make this divisible by four.

int npart = 0;      // Number of particles in the simulation

int maxmemb = 8;    // Maximum number of particles per bottom level cluster

int nxnodes = 129;  // Set the grid size for sampling the potential

int usegq = 0;      // Set the method for panel calculation

int invert = 1;     // Set whether or not to invert the panel effects matrix

// If zero, then GMRES is used to solve for

// panel strengths

double dlength;   // Domain length

double dheight;   // Domain height

double xfor;      // Temporary x-force

double yfor;      // Temporary y-force

double clocktime; // Time computed using clock cycles

double dx;

double density = 1e13; // Particle number density

time_t beg;       // Beginning time (in seconds)

time_t end;       // Ending time (in seconds)

clock_t begtime;  // Beginning time (in clock cycles)

clock_t endtime;  // Ending time (in clock cycles)

double* panelarray;     // Panel effects array

PANEL* pan = NULL;      // Panel array

PARTICLE* part = NULL;  // Particle array

// Initialize variables

del=1e-6;

usegq = 0;    // Indicate whether or not to use GQ panel integration

npoints = 8;  // Number of GQ points on each panel

// Domain and panel setup

dlength = 0.1;      // Set the domain length

dheight = 0.1;      // Set the domain height

numpan = 128;       // Set the number of panels

side = numpan / 4;  // Set the number of panels on each side.

npart = 10000;      // Set the number of particles

nxnodes = 129;      // Set the number of nodes for potential field sampling

density = 1.0e13;   // Set the particle number density

// Read DS input file

//ifstream in("ds-input.txt");

//in >> nxnodes;

//in >> del;

//in >> numpan;

//in.close();

side = numpan / 4;

dx = dlength / static_cast<double>(nxnodes);

/////////////////////////////////////////////

// Array allocation

part = new PARTICLE[npart];             // Allocate the particle array

pan = new PANEL[numpan];                // Allocate the panel array

panelarray = new double[numpan*numpan]; // Allocate the panel effect array

/////////////////////////////////////////////

// Domain setup functions

// Set up the boundaries and panels

cout << "Setting up boundaries" << endl;

setboundaries(side, side, side, dlength, dheight, pan, 0.0, usegq);

// Initialize the particles in the domain

cout << "Initializing particles" << endl;

boxparticles(npart, dlength, dheight, part, density);

// Compute the panel effects array

cout << "Computing panel effects array" << endl;

panelconstantsreal(numpan, pan, panelarray, invert);

cout << "Panel array construction and inversion done." << endl;

time(&beg);       // Get the initial times

begtime=clock();

/////////////////////////////////////////////

// Main loop

ofstream log("log.txt");

log << "Direct Summation" << endl;

log << "del = " << del << endl;

log << "numpan = " << numpan << endl;

// Solve for the panel strengths

panelsolvereal(numpan, npart, panelarray, part, pan, invert);

// Compute the particle forces

for (i=0; i<npart; i++)  {

// Force due to the panels

efield_panels(part[i].x, part[i].y, &part[i].xforce,

&part[i].yforce, numpan, pan, usegq);

// Use the treecode to compute the force

xfor=0.0;

yfor=0.0;

forceparticlereal(i, npart, &xfor, &yfor, part);

part[i].xforce+=xfor*charge_constant;

part[i].yforce+=yfor*charge_constant;

// Multiply by the particle weight

part[i].xforce*=part[i].tot_charge;

part[i].yforce*=part[i].tot_charge;

}

/////////////////////////////////////////////

// Calculate the total time in seconds

// For very long cases, this will always be correct.

endtime=clock();    // Get the ending times

time(&end);

clocktime=(endtime-begtime)/static_cast<double>(CLOCKS_PER_SEC);

cout << clocktime << "\t\t" << static_cast<int>(end-beg) << endl;

log << clocktime << "\t\t" << static_cast<int>(end-beg) << endl;

log.close();

// Output to files

cout << "Outputting to files" << endl;

output_particles(npart, part, "ds-partforces.txt");

plotpot(nxnodes, numpan, npart, 0, 0, 0.0, dlength, dheight, pan,

part, NULL, usegq);

plot_efield_ds(nxnodes, dlength, npart, numpan, part, pan, usegq);

delete [] pan;        // Delete the panel array

delete [] panelarray; // Delete the panel effects array

delete [] part;       // Delete the particle array

delete [] boundaries; // Delete the boundary array

return(0);

*/

//}

// Set up boundaries in the box domain

int setboundaries(int toppan, int botpan, int sidepan, double dlength,

double dheight, PANEL* pan, double maxpot, int usegq)  {

int i=0;

int dirichlet=0;

int neumann=1;

int totpan=0;

int ind=0;

int toptype=neumann;

int bottype=neumann;

int rtype=dirichlet;

int ltype=dirichlet;

double phitopref=0.0;

double phibotref=0.0;

double phirightref=0.0;

double phileftref=5.0e4;

double temp=0.0;

double dx;

double dy;

double* gpts = NULL;

boundary_points = 5;

boundaries = new BOUNDARY[boundary_points];

i=0;

boundaries[i].x0=0.0;

boundaries[i].y0=0.0;

boundaries[i].x_normal=1.0;

boundaries[i].y_normal=0.0;

boundaries[i].type=ltype;

boundaries[i].refpot=phileftref;

boundaries[i].length=dheight;

boundaries[i].direction=1;

i=1;

boundaries[i].x0=0.0;

boundaries[i].y0=dheight;

boundaries[i].x_normal=0.0;

boundaries[i].y_normal=-1.0;

boundaries[i].type=toptype;

boundaries[i].refpot=phitopref;

boundaries[i].length=dlength;

boundaries[i].direction=2;

i=2;

boundaries[i].x0=dlength;

boundaries[i].y0=dheight;

boundaries[i].x_normal=-1.0;

boundaries[i].y_normal=0.0;

boundaries[i].type=rtype;

boundaries[i].refpot=phirightref;

boundaries[i].length=dheight;

boundaries[i].direction=1;

i=3;

boundaries[i].x0=dlength;

boundaries[i].y0=0.0;

boundaries[i].x_normal=0.0;

boundaries[i].y_normal=1.0;

boundaries[i].type=bottype;

boundaries[i].refpot=phibotref;

boundaries[i].length=dlength;

boundaries[i].direction=2;

i=4;

boundaries[i].x0=0.0;

boundaries[i].y0=0.0;

boundaries[i].x_normal=0.0;

boundaries[i].y_normal=1.0;

boundaries[i].type=bottype;

boundaries[i].refpot=phibotref;

boundaries[i].length=dlength;

boundaries[i].direction=2;

// Using Gaussian Quadrature for panel integration

if (usegq)  {

// Declare arrays for weights and points

gpts=new double[npoints];

wts=new double[npoints];

gaussvals(npoints, gpts, wts);    // Get the GQ points and weights

}

double* x;

x=new double[sidepan+1];

temp=pi/(sidepan-1.0);

x[0]=0.0;

for (i=1; i<sidepan; i++)

x[i]=dlength*0.5*(1.0-cos((i-0.5)*temp));

x[sidepan]=dlength;

// Set up the left side panels

dx=0.0;                   // Set the change in x

dy=dheight/sidepan;  // Set the change in y

for (i=0; i<sidepan; i++)  {

ind=i+totpan;           // Increment the panel index

// Initialize the panel properties

panelinit(&pan[ind], 0.0, 0.0, x[i], x[i+1], phileftref, 1.0, 0.0,

ltype, gpts, usegq);

}

totpan+=sidepan;          // Increment the total number of panels so far

// Set up the top panels

dx=dlength/toppan;

dy=0.0;

for (i=0; i<toppan; i++)  {

ind=i+totpan;

panelinit(&pan[ind], x[i], x[i+1], dheight, dheight, phitopref,

0.0, -1.0, toptype, gpts, usegq);

}

totpan+=toppan;

// Set up the right side panels

dx=0.0;

dy=dheight/sidepan;

for (i=0; i<sidepan; i++)  {

ind=i+totpan;

panelinit(&pan[ind], dlength, dlength, dheight-x[i], dheight-x[i+1],

phirightref, -1.0, 0.0, rtype, gpts, usegq);

}

totpan+=sidepan;

// Set up the bottom panels

dx=dlength/toppan;

dy=0.0;

for (i=0; i<botpan; i++)  {

ind=i+totpan;

panelinit(&pan[ind], dlength-x[i], dlength-x[i+1], 0.0, 0.0,

phibotref, 0.0, 1.0, bottype, gpts, usegq);

}

totpan+=toppan;

if (usegq)

delete [] gpts;

delete [] x;

return(0);

}

/* // This is the original particle placement code. Created my own procedure to place the particles where I want them to go

// Create particles in the box domain

void boxparticles(int npart, double dlength, double dheight,

PARTICLE* part, double density)  {

int i;

int random = 1;   // Sets method for intializing particles

int nside;        // Number of particles per row or column if

// using even spacing

double teref = 0.1;

double mass = 9.1093897e-31;

double v0 = sqrt(8*teref*elec/(pi*mass));     // Thermal velocity

double beta = mass/(2*elec*teref);            // Maxwellian parameter

double rando=0.0;                             // Random number

double f=0.0;  // Fraction of Maxwellian

double v=0.0;  // Particle velocity

double angle;  // Velocity angle

double delta;  // Spacing between particles for even spacing

double weight = density * dlength * dheight / npart;  // Particle weight

double walldelta = 0.0;  // Distance to keep particles from the wall when

// setting positions randomly.

// Set the variables for evenly-spaced particles

if (!random)  {

nside = static_cast<int>(sqrt(static_cast<double>(npart)));

delta = dlength / nside;

}

else

walldelta = 0.0;

for (i=0; i<npart; i++)  {

part[i].charge = -1;

part[i].weight = weight;

// Random spacing

if (random)  {

part[i].x=walldelta + RandL()*(dlength - 2*walldelta);

part[i].y=walldelta + RandL()*(dheight - 2*walldelta);

}

// Even spacing

else  {

part[i].x=(i%nside+0.5)*delta;

part[i].y=(i/nside+0.5)*delta;

}

// Sample the velocity from a Maxwellian

f=0.0;

while (f < RandL())  {

v=v0*(3.0-6.0*RandL());

f=exp(-beta*v*v);

}

angle=pi*RandL();

part[i].u=v*cos(angle);

part[i].v=v*sin(angle);

part[i].tot_charge=part[i].charge*part[i].weight*elec;

// Set the particles to be movable (start off with all electrons) Is this ok, or should I start with equal numbers electrons and ions?

part[i].movable = true;

}

// Output the particle locations to file

//ofstream pt("partloc.txt");

//pt << "i\tx\ty" << endl;

//for (i=0; i<npart; i++)  {

//  pt << i << "\t" << part[i].x << "\t" << part[i].y << "\t"

//     << part[i].u << "\t" << part[i].v << "\t" << endl;

//

//}

//pt.close();

return;

}

*/

// Calculate the potential at a point using direct sum

double calcpot(double x, double y, int numpan, int npart,

PANEL* pan, PARTICLE* part, int usegq)  {

int i;

int not_on_boundary=1;

double pot=0.0;

double temp=0.0;

double rs;

double part_pot=0.0;

pot = potential_panel(x, y, numpan, pan, usegq);

for (i=0; i<npart; i++)  {

rs=(x-part[i].x)*(x-part[i].x)+(y-part[i].y)*(y-part[i].y);

if (rs > 1e-10)

part_pot-=log(rs)*part[i].tot_charge;

}

part_pot*=charge_constant*0.5;

pot+=part_pot;

return(pot);

}

// Calculate the potential at a point using the treecode

double calctreepot(double x, double y, int numpan, int npart, int nterms,

double acc, PANEL* pan, PARTICLE* part, TREE* tree,

int usegq)  {

double pot=0.0;

pot = potential_panel(x, y, numpan, pan, usegq);

pot -= treepot(0, nterms, x, y, acc, tree, part)*charge_constant;

return(pot);

}

// Compute the panel effects array using exact integration

int panelconstantsreal(int numpan, PANEL* pan, double* panelarray,

int invert)  {

int i;

int j;

int ind=0;

// Iterate through the panels to fill the array

for (i=0; i<numpan; i++)  {

if (pan[i].type)  {  //  ith panel is Neumann

for (j=0; j<numpan; j++)  {

panelarray[ind]=0.0;

if (pan[j].type)  {  //  jth panel is Neumann

// Panel j is horizontal

if (pan[j].xnrm == 0.0)  {

// Both panels are horizontal

if (pan[i].xnrm == 0.0)  {

panelarray[ind]=pan[i].ynrm*

arctan2(pan[j].x1, pan[j].x0, pan[i].midx,

pan[j].y0, pan[i].midy, pan[j].ynrm);

}

// Panel i is vertical

else  {

panelarray[ind]=pan[i].xnrm*0.5*

(lnrs(pan[j].x1,pan[i].midx,pan[j].y0,pan[i].midy)

-lnrs(pan[j].x0,pan[i].midx,pan[j].y0,pan[i].midy));

}

}

// Panel j is vertical

else  {

// Panel i is horizontal

if (pan[i].xnrm == 0.0)  {

panelarray[ind]=pan[i].ynrm*0.5*

(lnrs(pan[j].y1,pan[i].midy,pan[j].x0,pan[i].midx)

-lnrs(pan[j].y0,pan[i].midy,pan[j].x0,pan[i].midx));

}

// Both panels are vertical

else  {

panelarray[ind]=pan[i].xnrm*

arctan2(pan[j].y1, pan[j].y0, pan[i].midy,

pan[j].x0, pan[i].midx, pan[j].xnrm);

}

}

}

else  {  //  jth panel is Dirichlet

if (pan[j].xnrm == 0.0)  {

if (pan[i].xnrm==0.0)

panelarray[ind]=-pan[j].ynrm*pan[i].ynrm*

(invval(pan[j].x1,pan[i].midx,pan[j].y0,pan[i].midy)

-invval(pan[j].x0,pan[i].midx,pan[j].y0,pan[i].midy));

else

panelarray[ind]=pan[j].ynrm*pan[i].xnrm*

(invval(pan[j].y0,pan[i].midy,pan[j].x1,pan[i].midx)

-invval(pan[j].y0,pan[i].midy,pan[j].x0,pan[i].midx));

}

else  {

if (pan[i].xnrm==0.0)

panelarray[ind]=pan[j].xnrm*pan[i].ynrm*

(invval(pan[j].x0,pan[i].midx,pan[j].y1,pan[i].midy)

-invval(pan[j].x0,pan[i].midx,pan[j].y0,pan[i].midy));

else

panelarray[ind]=-pan[j].xnrm*pan[i].xnrm*

(invval(pan[j].y1,pan[i].midy,pan[j].x0,pan[i].midx)

-invval(pan[j].y0,pan[i].midy,pan[j].x0,pan[i].midx));

}

}

ind++;    // Increment the panelarray index

}

}

else  {  //  ith panel is Dirichlet

for (j=0; j<numpan; j++)  {

panelarray[ind]=0.0;

if (pan[j].type)  {  //  jth panel is Neumann

if (pan[j].xnrm == 0.0)  {

panelarray[ind]=

(logint(pan[j].x1,pan[i].midx,pan[j].y0,pan[i].midy)

-logint(pan[j].x0,pan[i].midx,pan[j].y0,pan[i].midy));

}

else  {

panelarray[ind]=

(logint(pan[j].y1,pan[i].midy,pan[j].x0,pan[i].midx)

-logint(pan[j].y0,pan[i].midy,pan[j].x0,pan[i].midx));

}

}

else  {  //  jth panel is Dirichlet

if (pan[j].xnrm == 0.0)  {

panelarray[ind]=pan[j].ynrm*

arctan2(pan[j].x1, pan[j].x0, pan[i].midx,

pan[j].y0, pan[i].midy, pan[j].ynrm);

}

else  {

panelarray[ind]=pan[j].xnrm*

arctan2(pan[j].y1, pan[j].y0, pan[i].midy,

pan[j].x0, pan[i].midx, pan[j].xnrm);

}

}

ind++;

}

}

}

// Output the panel effect array to file

/*ofstream lenout("realstrengths.txt");

for (i=0; i<numpan; i++)  {

for (j=0; j<numpan; j++)  {

lenout << panelarray[i*numpan+j] << "\t";

}

lenout << endl;

}

lenout.close();*/

// Invert the panel effects matrix if using that method

if (invert)

invertmatrix(panelarray, numpan);

return(0);

}

// Compute the panel effects array using Gaussian Quadrature

int panelconstantsgauss(int numpan, PANEL* pan, double* panelarray,

int invert)  {

int i;

int j;

int k;

int ind;

double temp;

double temp2;

double cnst=inv2pi*elec/perm;

double x;

double y;

double rs;

for (i=0; i<numpan; i++)  {

if (pan[i].type)  {  //  ith panel is Neumann

for (j=0; j<numpan; j++)  {

ind=i*numpan+j;

panelarray[ind]=0.0;

temp=0.0;

if (i==j)

panelarray[ind]=0.5*(pan[j].ynrm-pan[j].xnrm)/inv2pi;

else if (pan[j].type)  {  //  jth panel is Neumann

if (pan[j].xnrm == 0.0)  {

y=pan[i].midy-pan[j].midy;

temp2=y*y;

if (pan[i].xnrm == 0.0)  {

for (k=0; k<npoints; k++)  {

x=pan[i].midx-pan[j].points[k];

temp-=wts[k]/(x*x+temp2);

}

panelarray[ind]=pan[i].ynrm*y*temp*0.5*

(pan[j].x1-pan[j].x0);

}

else  {

for (k=0; k<npoints; k++)  {

x=pan[i].midx-pan[j].points[k];

temp-=wts[k]*x/(x*x+temp2);

}

panelarray[ind]=pan[i].xnrm*temp*0.5*

(pan[j].x1-pan[j].x0);

}

}

else  {

x=pan[i].midx-pan[j].midx;

temp2=x*x;

if (pan[i].xnrm == 0.0)  {

for (k=0; k<npoints; k++)  {

y=pan[i].midy-pan[j].points[k];

temp-=wts[k]*y/(temp2+y*y);

}

panelarray[ind]=pan[i].ynrm*temp*0.5*

(pan[j].y1-pan[j].y0);

}

else  {

for (k=0; k<npoints; k++)  {

y=pan[i].midy-pan[j].points[k];

temp-=wts[k]/(temp2+y*y);

}

panelarray[ind]=pan[i].xnrm*x*temp*0.5*

(pan[j].y1-pan[j].y0);

}

}

}

else  {  //  jth panel is Dirichlet

if (pan[j].xnrm == 0.0)  {

y=pan[i].midy-pan[j].midy;

temp2=y*y;

if (pan[i].xnrm==0.0)  {

for (k=0; k<npoints; k++)  {

x=pan[i].midx-pan[j].points[k];

rs=x*x+temp2;

temp-=wts[k]*(temp2-x*x)/(rs*rs);

}

panelarray[ind]=pan[i].ynrm*pan[j].ynrm*temp*0.5*

(pan[j].x1-pan[j].x0);

}

else  {

for (k=0; k<npoints; k++)  {

x=pan[i].midx-pan[j].points[k];

rs=x*x+temp2;

temp-=wts[k]*x/(rs*rs);

}

panelarray[ind]=pan[i].xnrm*pan[j].ynrm*y*temp*

(pan[j].x1-pan[j].x0);

}

}

else  {

x=pan[i].midx-pan[j].midx;

temp2=x*x;

if (pan[i].xnrm==0.0)  {

for (k=0; k<npoints; k++)  {

y=pan[i].midy-pan[j].points[k];

rs=temp2+y*y;

temp-=wts[k]*y/(rs*rs);

}

panelarray[ind]=pan[i].ynrm*pan[j].xnrm*x*temp*

(pan[j].y1-pan[j].y0);

}

else  {

for (k=0; k<npoints; k++)  {

y=pan[i].midy-pan[j].points[k];

rs=temp2+y*y;

temp-=wts[k]*(temp2-y*y)/(rs*rs);

}

panelarray[ind]=pan[i].xnrm*pan[j].xnrm*temp*0.5*

(pan[j].y1-pan[j].y0);

}

}

}

}

}

else  {  //  ith panel is Dirichlet

for (j=0; j<numpan; j++)  {

ind=i*numpan+j;

panelarray[ind]=0.0;

temp=0.0;

if (i==j)

panelarray[ind]=0.5*(pan[j].ynrm-pan[j].xnrm)/inv2pi;

else if (pan[j].type)  {  //  jth panel is Neumann

if (pan[j].xnrm == 0.0)  {

y=pan[i].midy-pan[j].midy;

temp2=y*y;

for (k=0; k<npoints; k++)  {

x=pan[i].midx-pan[j].points[k];

temp+=wts[k]*log(x*x+temp2);

}

panelarray[ind]=0.25*temp*(pan[j].x1-pan[j].x0);

}

else  {

x=pan[i].midx-pan[j].midx;

temp2=x*x;

for (k=0; k<npoints; k++)  {

y=pan[i].midy-pan[j].points[k];

temp+=wts[k]*log(temp2+y*y);

}

panelarray[ind]=0.25*temp*(pan[j].y1-pan[j].y0);

}

}

else  {  //  jth panel is Dirichlet

if (pan[j].xnrm == 0.0)  {

y=pan[i].midy-pan[j].midy;

temp2=y*y;

for (k=0; k<npoints; k++)  {

x=pan[i].midx-pan[j].points[k];

temp-=wts[k]/(x*x+temp2);

}

panelarray[ind]=pan[j].ynrm*y*temp*0.5*

(pan[j].x1-pan[j].x0);

}

else  {

x=pan[i].midx-pan[j].midx;

temp2=x*x;

for (k=0; k<npoints; k++)  {

y=pan[i].midy-pan[j].points[k];

temp-=wts[k]/(temp2+y*y);

}

panelarray[ind]=pan[j].xnrm*x*temp*0.5*

(pan[j].y1-pan[j].y0);

}

}

}

}

}

/*ofstream lenout("gaussstrengths.txt");

for (i=0; i<numpan; i++)  {

for (j=0; j<numpan; j++)  {

lenout << panelarray[i*numpan+j] << "\t";

}

lenout << endl;

}

lenout.close();*/

// Invert the panel effects matrix if using that method

if (invert)

invertmatrix(panelarray, numpan);

return(0);

}

// Solve for the panel strengths using direct sum

int panelsolvereal(int numpan, int npart, double* panelarray,

PARTICLE* part, PANEL* pan, int invert)  {

int i;

int j;

int ind;

double x;

double y;

double rs;

double* gammas;

double* phis;

gammas=new double[numpan];

phis=new double[numpan];

if (invert)  {

for (i=0; i<numpan; i++)  {

phis[i]=pan[i].refpot;

gammas[i]=0.0;

}

}

else  {

for (i=0; i<numpan; i++)  {

phis[i]=pan[i].refpot;

gammas[i]=pan[i].str;

}

}

// Get the particle contributions at each panel center

for (i=0; i<numpan; i++)  {

if (pan[i].type)  {  //  ith panel is Neumann

for (j=0; j<npart; j++)  {

x=pan[i].midx-part[j].x;

y=pan[i].midy-part[j].y;

rs=x*x+y*y;

if (rs==0.0)

cout << "Error: Particle has zero distance from panel center : "

<< j << endl;

if (pan[i].xnrm == 0.0)

phis[i]-=charge_constant*y*pan[i].ynrm*part[j].tot_charge/rs;

else

phis[i]-=charge_constant*x*pan[i].xnrm*part[j].tot_charge/rs;

}

}

else  {  //  ith panel is Dirichlet

for (j=0; j<npart; j++)  {

x=pan[i].midx-part[j].x;

y=pan[i].midy-part[j].y;

rs=x*x+y*y;

if (rs==0.0)

cout << "Error: Particle has zero distance from panel center : "

<< j << endl;

phis[i]+=charge_constant*0.5*log(rs)*part[j].tot_charge;

}

}

}

// If the panel effects matrix was inverted, just multiply by the phi array

if (invert)  {

ind=0;

for (i=0; i<numpan; i++)  {

for (j=0; j<numpan; j++)  {

gammas[i]+=panelarray[ind]*phis[j];

ind++;

}

}

}

// If there was no inversion, solve using GMRES

else

gmresclean(numpan, panelarray, phis, gammas);

// Assign panel strengths

for (i=0; i<numpan; i++)

pan[i].str=gammas[i];

// Output panel strengths

/*ofstream gam("realgammasphis.txt");

gam << "Index\tPhi\tGamma" << endl;

for (i=0; i<numpan; i++)

gam << i << "\t" << phis[i] << "\t" << gammas[i] << endl;

gam.close();*/

// Clean up arrays

delete [] phis;

delete [] gammas;

return(0);

}

// Solve for the panel strengths using the treecode and matrix inversion

int panelsolvetree(int numpan, int npart, int nterms, double acc,

double* panelarray, PARTICLE* part, PANEL* pan,

TREE* tree, int invert)  {

int i;

int j;

int ind;

double xfor;

double yfor;

double* gammas;

double* phis;

gammas=new double[numpan];

phis=new double[numpan];

if (invert)  {

for (i=0; i<numpan; i++)  {

phis[i]=pan[i].refpot;

gammas[i]=0.0;

}

}

else  {

for (i=0; i<numpan; i++)  {

phis[i]=pan[i].refpot;

gammas[i]=pan[i].str;

}

}

// Compute the particle effects on panels using the treecode

for (i=0; i<numpan; i++)  {

if (pan[i].type)  {  //  ith panel is Neumann

xfor=0.0;

yfor=0.0;

treeforce(0, -1, nterms, pan[i].midx, pan[i].midy, acc, &xfor, &yfor,

tree, part);

phis[i]-=charge_constant*(xfor*pan[i].xnrm+yfor*pan[i].ynrm);

}

else  {  //  ith panel is Dirichlet

phis[i]+=charge_constant*treepot(0, nterms, pan[i].midx, pan[i].midy,

acc, tree, part);

}

}

// If the panel effects matrix was inverted, just multiply by the phi array

if (invert)  {

ind=0;

for (i=0; i<numpan; i++)  {

for (j=0; j<numpan; j++)  {

gammas[i]+=panelarray[ind]*phis[j];

ind++;

}

}

}

// If there was no inversion, solve using GMRES

else

gmresclean(numpan, panelarray, phis, gammas);

for (i=0; i<numpan; i++)

pan[i].str=gammas[i];

/*ofstream gam("treegammasphis.txt");

gam << "Index\tPhi\tGamma" << endl;

for (i=0; i<numpan; i++)

gam << i << "\t" << phis[i] << "\t" << gammas[i] << endl;

gam.close();*/

delete [] phis;

delete [] gammas;

return(0);

}

// Direct sum particle force computation

// This skips the particle that is the input

void forceparticlereal(int ind, int npart, double* xfor,

double* yfor, PARTICLE* part)  {

int i;

double temp=0.0;

double x_force=0.0;

double y_force=0.0;

double dx;

double dy;

for (i=0; i<npart; i++)  {

if (i != ind)  {

dx=part[i].x-part[ind].x;

dy=part[i].y-part[ind].y;

temp=part[i].tot_charge/(dx*dx+dy*dy+del);

x_force-=dx*temp;

y_force-=dy*temp;

}

}

*xfor = x_force;

*yfor = y_force;

}

void forceparticlereal_point(double x, double y, int npart, double* xfor,

double* yfor, PARTICLE* part)  {

int i;

double temp=0.0;

double x_force=0.0;

double y_force=0.0;

double dx;

double dy;

for (i=0; i<npart; i++)  {

dx=part[i].x-x;

dy=part[i].y-y;

temp=part[i].tot_charge/(dx*dx+dy*dy+del);

x_force-=dx*temp;

y_force-=dy*temp;

}

*xfor = x_force;

*yfor = y_force;

}

// Direct sum electric field computation at point

// This includes all particles

void efield_particles(double x, double y, double* px_force, double* py_force,

int npart, PARTICLE* part)  {

int i;

double temp=0.0;

double xfor=0.0;

double yfor=0.0;

double dx;

double dy;

for (i=0; i<npart; i++)  {

dx=part[i].x-x;

dy=part[i].y-y;

temp=dx*dx+dy*dy;

if (temp > eps)  {

temp=part[i].tot_charge/(temp+del);

xfor-=dx*temp;

yfor-=dy*temp;

}

}

*px_force+=xfor*charge_constant;

*py_force+=yfor*charge_constant;

}

// Compute the force from the panels on a single particle

void forcepanelreal(int numpan, PARTICLE* ppart, PANEL* pan)  {

int i=0;

double xfor=0.0;

double yfor=0.0;

double r1=0.0;

double r0=0.0;

double x1;

double y1;

double x0;

double y0;

// Iterate through the panels

for (i=0; i<numpan; i++)  {

// Neumann panel

if (pan[i].type)  {

// Panel is horizontal

if (pan[i].xnrm==0.0)  {

y0=pan[i].y0-ppart->y; // y-position

x1=pan[i].x1-ppart->x; // Change in x from panel endpoint

x0=pan[i].x0-ppart->x; // Change in x from panel beginning

r1=y0*y0+x1*x1;       // Distance to endpoint

r0=y0*y0+x0*x0;       // Distance to beginning

// Compute the forces via direct integration

xfor+=pan[i].str*0.5*(log(r1)-log(r0));

yfor+=pan[i].str*(

arctan(pan[i].x1, ppart->x, pan[i].y1, ppart->y, pan[i].ynrm)-

arctan(pan[i].x0, ppart->x, pan[i].y0, ppart->y, pan[i].ynrm));

}

// Panel is vertical

else  {

x0=pan[i].x0-ppart->x; // x-position

y1=pan[i].y1-ppart->y; // Change in y from panel endpoint

y0=pan[i].y0-ppart->y; // Change in y from panel beginning

r1=y1*y1+x0*x0;       // Distance to endpoint

r0=y0*y0+x0*x0;       // Distance to beginning

// Compute the forces via direct integration

xfor+=pan[i].str*(

arctan(pan[i].y1, ppart->y, pan[i].x1, ppart->x, pan[i].xnrm)-

arctan(pan[i].y0, ppart->y, pan[i].x0, ppart->x, pan[i].xnrm));

yfor+=pan[i].str*0.5*(log(r1)-log(r0));

}

}

// Dirichlet panel

else  {

// Panel is horizontal

if (pan[i].xnrm==0.0)  {

xfor+=pan[i].str*pan[i].ynrm*

(invval(pan[i].y1, ppart->y, pan[i].x1, ppart->x)

-invval(pan[i].y0, ppart->y, pan[i].x0, ppart->x));

yfor+=-pan[i].str*pan[i].ynrm*

(invval(pan[i].x1, ppart->x, pan[i].y1, ppart->y)

-invval(pan[i].x0, ppart->x, pan[i].y0, ppart->y));

}

// Panel is vertical

else  {

xfor+=-pan[i].str*pan[i].xnrm*

(invval(pan[i].y1, ppart->y, pan[i].x1, ppart->x)

-invval(pan[i].y0, ppart->y, pan[i].x0, ppart->x));

yfor+=pan[i].str*pan[i].xnrm*

(invval(pan[i].x1, ppart->x, pan[i].y1, ppart->y)

-invval(pan[i].x0, ppart->x, pan[i].y0, ppart->y));

}

}

}

// Multiply by the particle's charge and weight

xfor*=ppart->tot_charge;

yfor*=ppart->tot_charge;

// Add in the force

ppart->xforce+=xfor;

ppart->yforce+=yfor;

return;

}

// Calculate the potential at a point from the panels only

double potential_panel(double x, double y, int numpan, PANEL* pan, int usegq)  {

int i;

double pot=0.0;

// Integrate normally

if (!usegq)  {

for (i=0; i<numpan; i++)  {

if (pan[i].type)  {  //  ith panel is Neumann

if (pan[i].xnrm == 0.0)  {

pot+=pan[i].str*

(logint(pan[i].x1, x, pan[i].y0, y)

-logint(pan[i].x0, x, pan[i].y0, y));

}

else  {

pot+=pan[i].str*

(logint(pan[i].y1, y, pan[i].x0, x)

-logint(pan[i].y0, y, pan[i].x0, x));

}

}

else  {  //  ith panel is Dirichlet

if (pan[i].xnrm == 0.0)  {

pot+=pan[i].str*pan[i].ynrm*

arctan2(pan[i].x1, pan[i].x0, x, pan[i].y0, y, pan[i].ynrm);

}

else  {

pot+=pan[i].str*pan[i].xnrm*

arctan2(pan[i].y1, pan[i].y0, y, pan[i].x0, x, pan[i].xnrm);

}

}

}

}

// Use Gaussian Quadrature to integrate the panels

else  {

int k;

double temp=0.0;

double temp2;

double dx;

double dy;

for (i=0; i<numpan; i++)  {

temp=0.0;

if (pan[i].type)  {  //  ith panel is Neumann

if (pan[i].xnrm == 0.0)  {

dy=y-pan[i].midy;

temp2=dy*dy;

for (k=0; k<npoints; k++)  {

dx=x-pan[i].points[k];

temp+=wts[k]*log(dx*dx+temp2);

}

pot+=0.25*pan[i].str*temp*(pan[i].x1-pan[i].x0);

}

else  {

dx=x-pan[i].midx;

temp2=dx*dx;

for (k=0; k<npoints; k++)  {

dy=y-pan[i].points[k];

temp+=wts[k]*log(temp2+dy*dy);

}

pot+=0.25*pan[i].str*temp*(pan[i].y1-pan[i].y0);

}

}

else  {  //  ith panel is Dirichlet

if (pan[i].xnrm == 0.0)  {

dy=y-pan[i].midy;

temp2=dy*dy;

for (k=0; k<npoints; k++)  {

dx=x-pan[i].points[k];

temp-=wts[k]/(dx*dx+temp2);

}

pot+=pan[i].str*pan[i].ynrm*dy*temp*0.5*

(pan[i].x1-pan[i].x0);

}

else  {

dx=x-pan[i].midx;

temp2=dx*dx;

for (k=0; k<npoints; k++)  {

dy=y-pan[i].points[k];

temp-=wts[k]/(temp2+dy*dy);

}

pot+=pan[i].str*pan[i].xnrm*dx*temp*0.5*

(pan[i].y1-pan[i].y0);

}

}

}

}

return(pot);

}

// Compute the force from the panels on a point

void force_panel(double x, double y, double* px_force, double* py_force,

int numpan, PANEL* pan)  {

int i=0;

double x_force=0.0;

double y_force=0.0;

double r1=0.0;

double r0=0.0;

double x1;

double y1;

double x0;

double y0;

// Iterate through the panels

for (i=0; i<numpan; i++)  {

// Neumann panel

if (pan[i].type)  {

// Panel is horizontal

if (pan[i].xnrm==0.0)  {

y0=pan[i].y0-y; // y-position

x1=pan[i].x1-x; // Change in x from panel endpoint

x0=pan[i].x0-x; // Change in x from panel beginning

r1=y0*y0+x1*x1; // Distance to endpoint

r0=y0*y0+x0*x0; // Distance to beginning

// Compute the forces via direct integration

x_force+=pan[i].str*0.5*(log(r1)-log(r0));

y_force+=pan[i].str*

arctan2(pan[i].x1, pan[i].x0, x, pan[i].y0, y, pan[i].ynrm);

}

// Panel is vertical

else  {

x0=pan[i].x0-x; // x-position

y1=pan[i].y1-y; // Change in y from panel endpoint

y0=pan[i].y0-y; // Change in y from panel beginning

r1=y1*y1+x0*x0;       // Distance to endpoint

r0=y0*y0+x0*x0;       // Distance to beginning

// Compute the forces via direct integration

x_force+=pan[i].str*

arctan2(pan[i].y1, pan[i].y0, y, pan[i].x0, x, pan[i].xnrm);

y_force+=pan[i].str*0.5*(log(r1)-log(r0));

}

}

// Dirichlet panel

else  {

// Panel is horizontal

if (pan[i].xnrm==0.0)  {

x_force+=pan[i].str*pan[i].ynrm*

(invval(pan[i].y1, y, pan[i].x1, x)

-invval(pan[i].y0, y, pan[i].x0, x));

y_force+=-pan[i].str*pan[i].ynrm*

(invval(pan[i].x1, x, pan[i].y1, y)

-invval(pan[i].x0, x, pan[i].y0, y));

}

// Panel is vertical

else  {

x_force+=-pan[i].str*pan[i].xnrm*

(invval(pan[i].y1, y, pan[i].x1, x)

-invval(pan[i].y0, y, pan[i].x0, x));

y_force+=pan[i].str*pan[i].xnrm*

(invval(pan[i].x1, x, pan[i].y1, y)

-invval(pan[i].x0, x, pan[i].y0, y));

}

}

}

*px_force=x_force;

*py_force=y_force;

return;

}

// Compute the panel force on a particle using Gaussian Quadrature

void force_panel_GQ(double xp, double yp, double* px_force, double* py_force,

int numpan, PANEL* pan)  {

int i;

int k;

double xfor=0.0;

double yfor=0.0;

double rs=0.0;

double r0=0.0;

double x;

double y;

double ytem;

double xtem;

double temp;

double temp2;

double walldelta=0.0;

double rlimit=walldelta*walldelta;

for (i=0; i<numpan; i++)  {

rs=(xp-pan[i].midx)*(xp-pan[i].midx)+

(yp-pan[i].midy)*(yp-pan[i].midy);

if (rs < rlimit)  {

if (pan[i].xnrm == 0.0)

yp=pan[i].midy+pan[i].ynrm*walldelta;

else

xp=pan[i].midx+pan[i].xnrm*walldelta;

}

}

for (i=0; i<numpan; i++)  {

xtem=0.0;

ytem=0.0;

if (pan[i].type)  {    //  Neumann panel

if (pan[i].xnrm == 0.0)  {  //  Panel is oriented in the x-direction

y=yp-pan[i].midy;

temp2=y*y;

for (k=0; k<npoints; k++)  {

x=xp-pan[i].points[k];

temp=wts[k]/(x*x+temp2);

xtem-=temp*x;

ytem-=temp;

}

xfor+=xtem*pan[i].str*0.5*(pan[i].x1-pan[i].x0);

yfor+=y*ytem*pan[i].str*0.5*(pan[i].x1-pan[i].x0);

}

else  {  //  Panel is oriented in the y-direction

x=xp-pan[i].midx;

temp2=x*x;

for (k=0; k<npoints; k++)  {

y=yp-pan[i].points[k];

temp=wts[k]/(temp2+y*y);

xtem-=temp;

ytem-=y*temp;

}

xfor+=x*xtem*pan[i].str*0.5*(pan[i].y1-pan[i].y0);

yfor+=ytem*pan[i].str*0.5*(pan[i].y1-pan[i].y0);

}

}

else  {    //  Dirichlet Panel

if (pan[i].xnrm == 0.0)  {  //  x-oriented panel

y=yp-pan[i].midy;

temp2=y*y;

for (k=0; k<npoints; k++)  {

x=xp-pan[i].points[k];

rs=x*x+temp2;

temp=wts[k]/(rs*rs);

xtem-=x*temp;

ytem-=(temp2-x*x)*temp;

}

xfor+=pan[i].ynrm*2.0*y*xtem*pan[i].str*0.5*(pan[i].x1-pan[i].x0);

yfor+=pan[i].ynrm*ytem*pan[i].str*0.5*(pan[i].x1-pan[i].x0);

}

else  {  //  y-oriented panel

x=xp-pan[i].midx;

temp2=x*x;

for (k=0; k<npoints; k++)  {

y=yp-pan[i].points[k];

rs=temp2+y*y;

temp=wts[k]/(rs*rs);

xtem-=(temp2-y*y)*temp;

ytem-=y*temp;

}

xfor+=pan[i].xnrm*temp*pan[i].str*0.5*(pan[i].y1-pan[i].y0);

yfor+=pan[i].xnrm*2.0*x*temp*pan[i].str*0.5*(pan[i].y1-pan[i].y0);

}

}

}

*px_force=xfor;

*py_force=yfor;

return;

}

// Initialize a panel using the inputs

void panelinit(PANEL* panl, double x0, double x1, double y0, double y1,

double refpot, double xnrm, double ynrm, int type,

double* gpts, int usegq)  {

panl->x0=x0;

panl->x1=x1;

panl->y0=y0;

panl->y1=y1;

panl->midx=0.5*(x1+x0);

panl->midy=0.5*(y1+y0);

panl->refpot=refpot;

panl->xnrm=xnrm;

panl->ynrm=ynrm;

panl->type=type;

panl->str=0.0;

// Using Gaussian Quadrature

if (usegq)  {

panl->points=new double[npoints];

int i;

// Initialize the GQ points on the panel

if (xnrm==0.0)  {

for (i=0; i<npoints; i++)

panl->points[i]=0.5*(x1+x0+gpts[i]*(x1-x0));

}

else  {

for (i=0; i<npoints; i++)

panl->points[i]=0.5*(y1+y0+gpts[i]*(y1-y0));

}

}

}

// Calculate the electric field at a point

int efield_panels(double x, double y, double* px_force, double* py_force,

int numpan, PANEL* pan, int usegq)  {

int x_bound=-1;

int y_bound=-1;

double p0;

double x_delta;

double y_delta;

double offset=0.01; // Determines when a point is "near" a boundary

double delta=0.001; // Size of the spacing when using standard differencing

double lower_limit=0.006;  // Indicates the size of the singular region in

// the potential calculation

double pot_min;

double pot_max;

offset = 0.00;

delta = 0.001;

lower_limit = 0.000;

// Neither coordinate is near the boundary - normal calculation

if (!nearboundary(x, y, offset, &x_bound, &y_bound))  {

if (usegq)

force_panel_GQ(x, y, px_force, py_force, numpan, pan);

else

force_panel(x, y, px_force, py_force, numpan, pan);

}

// At least one coordinate is near the boundary

else  {

// x is not near the boundary, so use standard differencing

if (x_bound == -1)  {

pot_min=potential_panel(x-delta, y, numpan, pan, usegq);

pot_max=potential_panel(x+delta, y, numpan, pan, usegq);

*px_force=(pot_min-pot_max)/(2.0*delta);

}

// x is close to the boundary

else  {

x_delta=fabs(x - boundaries[x_bound].x0);

// x is further than the "bad region"

if (x_delta > delta+lower_limit)  {

pot_min=potential_panel(x-delta, y, numpan, pan, usegq);

pot_max=potential_panel(x+delta, y, numpan, pan, usegq);

*px_force=(pot_min-pot_max)/(2.0*delta);

}

// x is close to or inside the "bad region"

else  {

if (boundaries[x_bound].x_normal > 0)

p0=boundaries[x_bound].x0+lower_limit;

else

p0=boundaries[x_bound].x0-lower_limit-delta-delta;

pot_min=potential_panel(p0, y, numpan, pan, usegq);

pot_max=potential_panel(p0+delta+delta, y, numpan, pan, usegq);

*px_force=(pot_min-pot_max)/(2.0*delta);

}

}

// y is not near the boundary, so use standard differencing

if (y_bound == -1)  {

pot_min=potential_panel(x, y-delta, numpan, pan, usegq);

pot_max=potential_panel(x, y+delta, numpan, pan, usegq);

*py_force=(pot_min-pot_max)/(2.0*delta);

}

// y is close to the boundary

else  {

y_delta=fabs(y - boundaries[y_bound].y0);

// y is further than the "bad region"

if (y_delta > delta+lower_limit)  {

pot_min=potential_panel(x, y-delta, numpan, pan, usegq);

pot_max=potential_panel(x, y+delta, numpan, pan, usegq);

*py_force=(pot_min-pot_max)/(2.0*delta);

}

// y is close to or inside the "bad region"

else  {

if (boundaries[y_bound].y_normal > 0)

p0=boundaries[y_bound].y0+lower_limit;

else

p0=boundaries[y_bound].y0-lower_limit-delta-delta;

pot_min=potential_panel(x, p0, numpan, pan, usegq);

pot_max=potential_panel(x,p0+delta+delta, numpan, pan, usegq);

*py_force=(pot_min-pot_max)/(2.0*delta);

}

}

}

return(0);

}

int nearboundary(double x, double y, double offset, int* px_bound, int* py_bound)  {

int i;

int nearb=0;

double delta=0.0;

for (i=0; i<boundary_points-1; i++)  {

// X-normal only

if (boundaries[i].direction == 1)  {

delta=fabs(x-boundaries[i].x0);

if (delta < offset)  {

nearb=1;

*px_bound=i;

}

}

// Y-normal only

else if (boundaries[i].direction == 2)  {

delta=fabs(y-boundaries[i].y0);

if (delta < offset)  {

nearb=1;

*py_bound=i;

}

}

}

return(nearb);

}

// Plot the potential field

int plotpot(int nxnodes, int numpan, int npart, int type, int nterms, double acc,

double dlength, double dheight, PANEL* pan,

PARTICLE* part, TREE* tree, int usegq)  {

int i=0;

int j=0;

int nc=nxnodes-1;

double x=0.0;

double y=0.0;

double dy=dheight/static_cast<double>(nc);

double dx=dlength/static_cast<double>(nc);

double* poten=new double[nxnodes*nxnodes];

string name;

char stemp[20];

// Using direct sum

if (type==0)  {

for (i=0; i<nxnodes; i++)  {

for (j=0; j<nxnodes; j++)  {

x=i*dx;

y=j*dy;

poten[i*nxnodes+j]=calcpot(x, y, numpan, npart, pan, part, usegq);

}

}

string name="pot-";

inttostr(numpan/4, stemp);

name+=stemp;

name+=".plt";

//ofstream ptpl(name.c_str());

ofstream ptpl("ds-potential.plt");

ptpl << "title='Potential'\nvariables=x,y,phi\n";

ptpl << "zone t=main, i=" << nxnodes << ", j=" << nxnodes << ", f=point\n";

for (j=0; j<nxnodes; j++)  {

y=j*dy;

for (i=0; i<nxnodes; i++)  {

x=i*dx;

ptpl << x << "\t" << y << "\t" << poten[i*nxnodes+j] << endl;

}

}

ptpl.close();

}

// Using the treecode

if (type==1)  {

for (i=0; i<nxnodes; i++)  {

for (j=0; j<nxnodes; j++)  {

x=i*dx;

y=j*dy;

poten[i*nxnodes+j]=calctreepot(x, y, numpan, npart, nterms, acc, pan,

part, tree, usegq);

}

}

ofstream trpt("tree-potential.plt");

trpt << "title='Potential'\nvariables=x,y,phi\n";

trpt << "zone t=main, i=" << nxnodes << ", j=" << nxnodes << ", f=point\n";

for (j=0; j<nxnodes; j++)  {

y=j*dy;

for (i=0; i<nxnodes; i++)  {

x=i*dx;

trpt << x << "\t" << y << "\t" << poten[i*nxnodes+j] << endl;

}

}

trpt.close();

}

delete [] poten;

return(0);

}

void plot_efield_tree(int nxnodes, int npart, int numpan, int nterms, int maxmemb,

double acc, double dlength, PARTICLE* part, PANEL* pan,

TREE* tree, int usegq)  {

int i;

int j;

double dx=dlength/(nxnodes-1.0);

double x=0.0;

double y=0.0;

double x_temp;

double y_temp;

double x_force;

double y_force;

double half=dlength*0.5;

ofstream file("tree-efield.plt");

file << "title='Electric Field'\nvariables=x,y,E_x,E_y\n";

file << "zone t=main, i=" << nxnodes << ", j=" << nxnodes << ", f=point\n";

for (i=0; i<nxnodes; i++)  {

x=0.0;

for (j=0; j<nxnodes; j++)  {

// Force due to the panels

efield_panels(x, y, &x_force, &y_force, numpan, pan, usegq);

x_temp=0.0;

y_temp=0.0;

// Use the treecode to compute the force

treeforce(0, -1, nterms, x, y, acc, &x_temp, &y_temp, tree, part);

x_force+=x_temp*charge_constant;

y_force+=y_temp*charge_constant;

file << x << "\t" << y << "\t" << x_force << "\t" << y_force << endl;

x+=dx;

}

y+=dx;

}

file.close();

return;

}

void plot_efield_ds(int nxnodes, double dlength, int npart, int numpan,

PARTICLE* part, PANEL* pan, int usegq)  {

int i;

int j;

double dx=dlength/(nxnodes-1.0);

double x=0.0;

double y=0.0;

double x_temp;

double y_temp;

double x_force;

double y_force;

double half=dlength*0.5;

ofstream file("ds-efield.plt");

file << "title='Electric Field'\nvariables=x,y,E_x,E_y\n";

file << "zone t=main, i=" << nxnodes << ", j=" << nxnodes << ", f=point\n";

for (i=0; i<nxnodes; i++)  {

x=0.0;

for (j=0; j<nxnodes; j++)  {

// Force due to the panels

efield_panels(x, y, &x_force, &y_force, numpan, pan, usegq);

x_temp=0.0;

y_temp=0.0;

// Use the treecode to compute the force

forceparticlereal_point(x, y, npart, &x_temp, &y_temp, part);

x_force+=x_temp*charge_constant;

y_force+=y_temp*charge_constant;

file << x << "\t" << y << "\t" << x_force << "\t" << y_force << endl;

x+=dx;

}

y+=dx;

}

file.close();

return;

}

void treeempty(int numtree, TREE* tree)  {

if (tree != NULL)  {

for (int i=0; i<numtree; i++)  {

if (tree[i].members != NULL)

delete [] tree[i].members;

tree[i].members=NULL;

if (tree[i].moments != NULL)

delete [] tree[i].moments;

tree[i].moments=NULL;

}

}

return;

}

// Generate the tree

int treemake(int npart, int maxmemb, double dlength, double dheight,

PARTICLE* part, TREE* tree)  {

int i=0;

int j=0;

int k=0;

int xcreate=0;

int ycreate=0;

int nchild=0;

int ind2=0;

int numtree=1;

// Top level tree dimensions

double minx=dlength;

double maxx=0.0;

double miny=dheight;

double maxy=0.0;

double aspectlimit=1.0/sqrt(2.0);   // Limit on the tree aspect ratio

// Initialize the top level tree

tree[0].nummemb=npart;

tree[0].members=new int[npart];

// Shrink the tree to fit the particles

for (i=0; i<npart; i++)  {

tree[0].members[i]=i;

if (minx > part[i].x)

minx=part[i].x;

if (maxx < part[i].x)

maxx=part[i].x;

if (miny > part[i].y)

miny=part[i].y;

if (maxy < part[i].y)

maxy=part[i].y;

}

treeinit(&tree[0], minx, maxx, miny, maxy);

i=-1;

// Compute the rest of the tree

while (i < numtree-1)  {

i++;

nchild=0;

/*if (i>=MAXCHILD-10)  {

cout << "Too many trees!" << i << endl;

cout << numtree << endl;

cin >> ch;

}*/

if (tree[i].nummemb > maxmemb)  {

// Division is needed

if (tree[i].aspect > aspectlimit)  {

// Divide in the x-direction

xcreate=treedivide(0, &tree[i], &tree[numtree],

&tree[numtree+1], part);

ind2=numtree+xcreate;

for (j=0; j<xcreate; j++)  {

// Add reference to new lower level tree

tree[i].children[nchild]=numtree+j;

nchild++;

if (tree[numtree+j].aspect < aspectlimit &&

tree[numtree+j].nummemb > maxmemb)  {

// Split the new tree in the y-direction

ycreate=treedivide(1, &tree[numtree+j], &tree[numtree+j],

&tree[ind2], part);

if (ycreate == 2)  {

tree[i].children[nchild]=ind2;

nchild++;

ind2++;

}

}

}

numtree+=nchild;    // Increment the total number of trees

}

else  {

// The aspect ratio is too small for x-division, so divide in y only

ycreate=treedivide(1, &tree[i], &tree[numtree],

&tree[numtree+1], part);

for (j=0; j<ycreate; j++)  {

tree[i].children[nchild]=numtree+j;

nchild++;

}

numtree+=nchild;

}

}

}

return(numtree);

}

// Initialize a tree cluster

void treeinit(TREE* tree1, double x0, double x1, double y0, double y1)  {

double x=0.5*(x1+x0);

double y=0.5*(y1+y0);

tree1->x0=x0;

tree1->x1=x1;

tree1->y0=y0;

tree1->y1=y1;

tree1->xmid=x;

tree1->ymid=y;

tree1->aspect=(x1-x0)/(y1-y0);

x=x1-x;

y=y1-y;

tree1->sqradius=x*x+y*y;

for (int i=0; i<NUMCHILD; i++)

tree1->children[i]=-1;

}

// Divide a tree cluster

// The split variable points to the tree being split, and the c1 and c2

// variables point to two new trees, if they are needed.

// split and c1 may point to the same tree

int treedivide(int dir, TREE* split, TREE* c1, TREE* c2, PARTICLE* part)  {

int i=0;

int j=0;

int k=0;

int n=0;

int ncreate=0;

int index;

int count[2];

double midpoint=0.0;

int* cmemb[2];

TREE* point;

// Get the number of particles that have to be assigned

n=split->nummemb;

cmemb[0]=new int[n];    // Allow enough space for all particles

cmemb[1]=new int[n];

// Divide in the x-direction

if (!dir)  {

count[0]=0;

count[1]=0;

midpoint=split->xmid;

// Count and index particles in each side

for (j=0; j<n; j++)  {

index = split->members[j];

if (part[index].x <= midpoint)  {

cmemb[0][count[0]] = index;

count[0]++;

}

else  {

cmemb[1][count[1]] = index;

count[1]++;

}

}

point=c1;

for (j=0; j<2; j++)  {

if (count[j] > 0)  {

// Assign particles on this side to the tree

point->nummemb=count[j];

if (point->members != NULL)

cout << "Error!  Non-null pointer!" << endl;

point->members=new int[count[j]];

for (k=0; k<count[j]; k++)

point->members[k]=cmemb[j][k];

treeshrink(point, part);

ncreate++;

point=c2;

}

}

}

// Divide in the y-direction

else  {

count[0]=0;

count[1]=0;

midpoint=split->ymid;

// Count and index particles in each side

for (j=0; j<n; j++)  {

index = split->members[j];

if (part[index].y <= midpoint)  {

cmemb[0][count[0]] = index;

count[0]++;

}

else  {

cmemb[1][count[1]] = index;

count[1]++;

}

}

point=c1;

for (j=0; j<2; j++)  {

if (count[j] > 0)  {

// Assign particles on this side to the tree

point->nummemb=count[j];

if (point->members != NULL)

delete [] point->members;

point->members=new int[count[j]];

for (k=0; k<count[j]; k++)

point->members[k]=cmemb[j][k];

treeshrink(point, part);

ncreate++;

point=c2;

}

}

}

// Clean up arrays

delete [] cmemb[0];

delete [] cmemb[1];

return(ncreate);    // Return the number of new trees created

}

// Shrink a tree to fit its member particle locations

void treeshrink(TREE* tree1, PARTICLE* part)  {

int j=0;

double maxx=0.0;

double maxy=0.0;

double minx=1e20;

double miny=1e20;

for (int i=0; i<tree1->nummemb; i++)  {

j=tree1->members[i];

if (part[j].x > maxx)

maxx=part[j].x;

if (part[j].x < minx)

minx=part[j].x;

if (part[j].y > maxy)

maxy=part[j].y;

if (part[j].y < miny)

miny=part[j].y;

}

treeinit(tree1, minx, maxx, miny, maxy);

}

// Calculate the Taylor expansion moments of the tree clusters

void treemoments(int numtree, int nterms, TREE* tree, PARTICLE* part)  {

int i;

int j;

int j1;

int k;

int ind;

double delx;

double dely;

double mult;

double cnst;

double ymul;

double* fact=new double[nterms];

// Compute the factorial terms

mult=1.0;

fact[0]=1.0;

if (nterms > 1)

fact[1]=1.0;

for (k=2; k<nterms; k++)  {

mult*=k;

fact[k]=mult;

}

for(ind=0; ind<numtree; ind++)  {

if (tree[ind].moments == NULL)

tree[ind].moments=new double[nterms*nterms];

// Initialize all moments to zero

for (k=0; k<nterms*nterms; k++)

tree[ind].moments[k]=0.0;

for(j1=0; j1<tree[ind].nummemb; j1++)  {

j=tree[ind].members[j1];

delx=part[j].x-tree[ind].xmid;

dely=part[j].y-tree[ind].ymid;

cnst=part[j].tot_charge;

mult=cnst;    // Initialize the x-moment tracker

ymul=cnst;    // Initialize the y-moment tracker

for (k=0; k<nterms; k++)  {

for (i=0; i<nterms-k; i++)  {

tree[ind].moments[i*nterms+k]+=mult;

mult*=delx;   // Increase the power of the x-moment

}

ymul*=dely;     // Increase the power of the y-moment

mult=ymul;      // Reset the total multiplier

}

}

// Divide by the factorial terms

for (k=0; k<nterms; k++)  {

for (i=0; i<k+1; i++)  {

j=k-i;

tree[ind].moments[i*nterms+j]/=fact[i]*fact[j];

}

}

}

delete [] fact;

return;

}

//  Calculate the force from each part of the tree on a particle p

double treeforce(int ind, int p, int nterms, double x, double y, double acc,

double* xfor, double* yfor, TREE* tree, PARTICLE* part)  {

int go=1;

int accept;

double dx=x-tree[ind].xmid;

double dy=y-tree[ind].ymid;

double rs=dx*dx+dy*dy;      // Determine the square of the radius

// Determine whether or not to accept the cluster

accept=(tree[ind].sqradius < acc*rs);

// On acceptance, use the Taylor expansion

if (accept)  {

treetaylorforce(nterms, dx, dy, rs+del, xfor, yfor, tree[ind].moments);

return(0);

}

// Cluster was not accepted

else  {

// Check for child clusters

for (int i=0; i<NUMCHILD; i++)  {

if (tree[ind].children[i] != -1)  {

go=0;

// Call the recursion

treeforce(tree[ind].children[i], p, nterms, x, y, acc, xfor, yfor,

tree, part);

}

}

// There are no child clusters, so do direct sum

if (go)  {

int j;

double temp=0.0;

// Do direct summation on all particles in the cell

if (p == -1)  {

for (int j1=0; j1<tree[ind].nummemb; j1++)  {

j=tree[ind].members[j1];

dx=x-part[j].x;

dy=y-part[j].y;

temp=dx*dx+dy*dy;

if (temp > eps)  {

temp+=del;

temp=part[j].tot_charge/temp;

*xfor+=dx*temp;

*yfor+=dy*temp;

}

}

}

else  {

for (int j1=0; j1<tree[ind].nummemb; j1++)  {

j=tree[ind].members[j1];

if (j != p)  {

dx=x-part[j].x;

dy=y-part[j].y;

temp=dx*dx+dy*dy;

if (temp > eps)  {

temp+=del;

temp=part[j].tot_charge/temp;

*xfor+=dx*temp;

*yfor+=dy*temp;

}

}

}

}

}

}

return(0);

}

// Uses the Taylor expansion to get the cluster force

void treetaylorforce(int nterms, double dx, double dy, double rs,

double* xfor, double* yfor, double* moment)  {

int i;

int j;

int k;

int k1;

int k2;

double invrs=1.0/rs;

double derivs[MAXTERM][MAXTERM];

derivs[0][0]=-0.5*log(rs);    // 0th order derivative

derivs[1][0]=dx/rs;            // 1st order derivatives

derivs[0][1]=dy/rs;

// 2nd order derivatives

derivs[2][0]=(derivs[1][0]*derivs[1][0]-derivs[0][1]*derivs[0][1]);

derivs[1][1]=2.0*derivs[1][0]*derivs[0][1];

derivs[0][2]=-derivs[2][0];

// Recursion relation for all higher order terms

for (k=3; k<nterms+1; k++)  {

k1=k-1;

k2=k-2;

derivs[k][0]=(2*k1*dx*derivs[k1][0]-k1*k2*derivs[k2][0])*invrs;

derivs[0][k]=(2*k1*dy*derivs[0][k1]-k1*k2*derivs[0][k2])*invrs;

derivs[k1][1]=(((2*k1-1)*dx*derivs[k1-1][1]-k2*k2*derivs[k-3][1])+

dy*derivs[k1][0])*invrs;

derivs[1][k1]=((2*k1-1)*dy*derivs[1][k1-1]-k2*k2*derivs[1][k-3]+

dx*derivs[0][k1])*invrs;

for (i=2; i<k1; i++)  {

j=k-i;

derivs[i][j]=((dx*(2*i-1)*derivs[i-1][j]-(i-1)*(i-1)*derivs[i-2][j])

+(dy*(2*j-1)*derivs[i][j-1]-(j-1)*(j-1)*derivs[i][j-2]))*invrs;

}

}

// Add up the Taylor derivatives and moments to get the force

for (k=0; k<nterms; k++)  {

for (i=k; i>-1; i--)  {

j=k-i;

*xfor+=moment[i*nterms+j]*derivs[i+1][j];

*yfor+=moment[i*nterms+j]*derivs[i][j+1];

}

}

return;

}

// Use the treecode to get the potential at a point

double treepot(int ind, int nterms, double x, double y, double acc,

TREE* tree, PARTICLE* part)  {

int i;

int j=0;

short int go=1;

short int accept=0;

double phi=0.0;

double dx=x-tree[ind].xmid;

double dy=y-tree[ind].ymid;

double rs=dx*dx+dy*dy;

accept=(tree[ind].sqradius/rs < acc);

if (accept)

return(treetaylorpot(nterms, dx, dy, rs, tree[ind].moments));

else  {

for (i=0; i<NUMCHILD; i++)  {

if (tree[ind].children[i] != -1)  {

go=0;

phi+=treepot(tree[ind].children[i], nterms, x, y, acc, tree, part);

}

}

if (go)  {

// Do direct summation on all particles in the cell

for (int j1=0; j1<tree[ind].nummemb; j1++)  {

j=tree[ind].members[j1];

phi+=part[j].tot_charge

*0.5*lnrs(x, part[j].x, y, part[j].y);

}

}

return(phi);

}

return(0);

}

// Use the Taylor expansion to get the cluster potential

double treetaylorpot(int nterms, double dx, double dy, double rs,

double* moment)  {

int j;

int i;

int k;

double invrs=1.0/rs;

double phi=0.0;

double derivs[MAXTERM][MAXTERM];

derivs[0][0]=-0.5*log(rs);

derivs[1][0]=dx*invrs;

derivs[0][1]=dy*invrs;

derivs[2][0]=(derivs[1][0]*derivs[1][0]-derivs[0][1]*derivs[0][1]);

derivs[1][1]=2.0*derivs[1][0]*derivs[0][1];

derivs[0][2]=-derivs[2][0];

int k1;

int k2;

for (k=3; k<nterms; k++)  {

k1=k-1;

k2=k-2;

derivs[k][0]=(2*k1*dx*derivs[k1][0]-k1*k2*derivs[k2][0])*invrs;

derivs[0][k]=(2*k1*dy*derivs[0][k1]-k1*k2*derivs[0][k2])*invrs;

derivs[k1][1]=(((2*k1-1)*dx*derivs[k1-1][1]-k2*k2*derivs[k-3][1])+

dy*derivs[k1][0])*invrs;

derivs[1][k1]=((2*k1-1)*dy*derivs[1][k1-1]-k2*k2*derivs[1][k-3]+

dx*derivs[0][k1])*invrs;

for (i=2; i<k1; i++)  {

j=k-i;

derivs[i][j]=((dx*(2*i-1)*derivs[i-1][j]-(i-1)*(i-1)*derivs[i-2][j])

+(dy*(2*j-1)*derivs[i][j-1]-(j-1)*(j-1)*derivs[i][j-2]))*invrs;

}

}

for (k=0; k<nterms; k++)  {

for (i=k; i>-1; i--)  {

j=k-i;

phi-=moment[i*nterms+j]*derivs[i][j];

}

}

return(phi);

}

void output_particles(int npart, PARTICLE* part, string filename)  {

int i;

// Output the particle properties to file

ofstream pfor(filename.c_str());

pfor << "i\tx\ty\tFx\tFy" << endl;

for (i=0; i<npart; i++)  {

pfor.precision(14);

pfor << i << "\t" << part[i].x << "\t" << part[i].y

<< "\t" << part[i].xforce << "\t" << part[i].yforce << endl;

}

pfor.close();

return;

}

// Calculate the value of alpha for a given e field magnitude. Piecewise linear fit to Pearl's data.

// Note that data is in the range of 1250 V/m to 9000000 V/m.

// If the field values get outside that range we might have some problems.

double calcalpha(double totefield) {

if (totefield < 1250.0)

return(6.7168e-93*(totefield-0));

if (totefield < 125000.0)

return(8.39600E-93+4.24888888888889E-18*(totefield - 1250));

if (totefield < 219600)

return(5.25800e-13+3.07266596194503e-15*(totefield - 125000));

if (totefield < 500000)

return(2.91200e-10+1.901243937232525e-12*(totefield - 219600));

if (totefield < 719000)

return(5.33400e-7+3.504383561643836e-11*(totefield - 500000));

if (totefield < 1048000)

return(8.20800e-6+2.817386018237081e-10*(totefield - 719000));

if (totefield < 2005200)

return(1.00900e-4+2.57323443376515e-9*(totefield - 1048000));

if (totefield < 3365400)

return(2.56400e-3+1.01720335244817e-8*(totefield - 2005200));

if (totefield < 4962200)

return(1.64000e-2+2.26640781563126e-8*(totefield - 3365400));

if (totefield < 6161300)

return(5.25900e-2+3.28329580518722e-8*(totefield - 4962200));

if (totefield < 6663800)

return(9.19600e-2+3.78905472636816e-8*(totefield - 6161300));

if (totefield < 7000000)

return(1.11000e-1+3.89649018441404e-8*(totefield - 6663800));

if (totefield < 8000000)

return(1.24100e-1+3.91000000000000e-8*(totefield - 7000000));

if (totefield < 9000000)

return(1.63200e-1+3.84000000000000e-8*(totefield - 8000000));

if (totefield >= 9000000)

return(0.2016);

}

// Calculate the value of mu for a given e field magnitude. This is from a quadratic fit to Pearl's data.

// Note that data is in the range of 1250 V/m to 9000000 V/m.

// If the field values get outside that range we might have some problems.

double calcmu(double totefield) {

if (totefield > 9e6)

cout << "E Field outside of range. Value: " << totefield << endl;

// Originally capped this at the range of the data that we had, but it's a good fit. R squared = 0.9986

// so hopefully  it's good outside the data range

/*

if (totefield > 9.0e6)

return 0.023859;

if (totefield < 1250.0)

return 0.38841;

*/

return(3.8751*pow(totefield, -0.3191));

}

// Add a particle with the specified parameters to the particle array at the end.

// Todo: Verify that this is not causing memory leaks... I don't know much about memory management in C++.

void addparticle(int& npart, PARTICLE*& part, double charge, double x, double y, double weight, bool movable, double dlength, double dheight) {

PARTICLE* temp = NULL;

int i;

temp = new PARTICLE[npart+1];

// Copy existing particles

for (i=0; i<npart; i++) {

temp[i] = part[i];

}

// Set properties

temp[npart].charge = charge;

temp[npart].x = x;

temp[npart].y = y;

temp[npart].weight = weight;

temp[npart].movable = movable;

// If we have an ion figure out which box it goes into

if (!movable) {

temp[npart].boxcol = getcol(x, dlength);

temp[npart].boxrow = getrow(y, dheight);

} else {

temp[npart].boxcol = -1;

temp[npart].boxrow = -1;

}

// Constant or calculated properties

temp[npart].u = 0.0; // My code doesn't use u and v, and ions don't move

temp[npart].v = 0.0; // My code doesn't use u and v, and ions don't move

temp[npart].xforce = 0.0; // Don't need to ever calculate force on ions

temp[npart].yforce = 0.0; // Don't need to ever claculate force on ions

temp[npart].tot_charge = temp[npart].charge*temp[npart].weight*elec;

temp[npart].alpha = 0.0; // Ions don't move, so don't need alpha

temp[npart].mu = 0.0; // Ions don't move, so don't need mu

delete [] part;

part = temp;

npart++;

};

// Add an ion to the particle array.

// This procedure will check to see if an ion is already in the box where the x and y coordinates want to add an ion.

// If there is an ion already, this procedure will modify the properties of that ion instead of adding a new ion.

// If there isn't an ion in the box already, this procedure will call addparticle with the ion's parameters.

// Todo: Verify that this is not causing memory leaks... I don't know much about memory management in C++.

void addion(int & npart, PARTICLE* & part, double charge, double x, double y, double weight, bool movable, double dlength, double dheight, int* & ion, int & nion) {

int i;

int col;

int row;

col = getcol(x,dlength);

row = getrow(y, dheight);

// Look through particles to find any that are in the same box as the new ion. OLD METHOD

/*

for (i=0; i<npart; i++) {

if (part[i].boxcol == col && part[i].boxrow == row && part[i].movable == false) {

// There is already an ion in this box

part[i].x = (part[i].weight*part[i].x + x*weight) / (part[i].weight + weight);

part[i].y = (part[i].weight*part[i].y + y*weight) / (part[i].weight + weight);

part[i].weight += weight;

part[i].tot_charge = part[i].charge*part[i].weight*elec;

return;

}

}

*/

// Look through particles to find any that are in the same box as the new ion. NEW METHOD

// Go backward through the ion array since new ion is likely to be near a recently added ion.

//for (i=0; i<nion; i++) { // Look forward through array

for (i=nion-1; i>=0; i--) { // Look backward through array

if (part[ion[i]].boxcol == col && part[ion[i]].boxrow == row) {

// There is already an ion in this box

part[ion[i]].x = (part[ion[i]].weight*part[ion[i]].x + x*weight) / (part[ion[i]].weight + weight);

part[ion[i]].y = (part[ion[i]].weight*part[ion[i]].y + y*weight) / (part[ion[i]].weight + weight);

part[ion[i]].weight += weight;

part[ion[i]].tot_charge = part[ion[i]].charge*part[ion[i]].weight*elec;

return;

}

}

// The box that the new ion is in does not contain any other ions

// Add the ion index to the ion array

int* temp = NULL;

temp = new int[nion+1];

for (i=0; i<nion; i++) {

temp[i] = ion[i];

}

temp[nion] = npart;

delete [] ion;

ion = temp;

nion++;

// Add the new ion to the particle array

addparticle(npart, part, charge, x, y, weight, movable, dlength, dheight);

return;

}

// Remove the specified particle from the particle array.

// Todo: Verify that this is not causing memory leaks... I don't know much about memory management in C++.

//

// This is commented out because it shouldn't be used, the simulation should be stopped before particles are removed

void removeparticle(int & npart, PARTICLE* & part, int partnum, int* & removedarray, int & nremoved) {

/*

PARTICLE* temp = NULL;

int* tempremoved = NULL;

int i;

if (partnum > npart-1 || partnum < 1) {

cout << "Invalid particle number!" << endl;

}

temp = new PARTICLE[npart-1];

// Copy existing particles

for (i=0; i<npart; i++) {

if (i!=partnum) {

temp[i] = part[i];

}

}

delete [] part;

part = temp;

npart--;

tempremoved = new int[nremoved + 1];

for (i=0; i<nremoved; i++) {

tempremoved[i] = removedarray[i];

}

tempremoved[nremoved] = partnum;

delete [] removedarray;

removedarray = tempremoved;

nremoved++;

*/

};

// Create particles in the box domain, modified to place them either randomly in a region or in a column evenly spaced.

void boxparticles(int npart, double dlength, double dheight,

PARTICLE* part, double density)  {

int i;

int random = 0;   // Sets method for intializing particles

//  int nside;        // Number of particles per row or column if

// using even spacing. Unneeded, now particles will be evenly spread in a column.

double xctr = 0.0075; // Center that the particles should be spaced around (x direction)

double yctr = 0.0001/2.0;  // Center that the particles should be spaced around (y direction)

double spread = 0.01; // Distance to spread the particles

double teref = 0.1;

double mass = 9.1093897e-31;

double v0 = sqrt(8*teref*elec/(pi*mass));     // Thermal velocity

double beta = mass/(2*elec*teref);            // Maxwellian parameter

double rando=0.0;                             // Random number

double delta;  // Spacing between particles for even spacing

double weight = density * dlength * dheight / npart;  // Particle weight

double walldelta = 0.0;  // Distance to keep particles from the wall when

// setting positions randomly.

// Set the variables for evenly-spaced particles

if (!random)  {

//nside = static_cast<int>(sqrt(static_cast<double>(npart)));

delta = dlength / npart;

}

else

walldelta = 0.0;

for (i=0; i<npart; i++)  {

// Set the particles to be movable (start off with all electrons)

part[i].charge = -1;

part[i].movable = true;

part[i].weight = weight;

part[i].tot_charge=part[i].charge*part[i].weight*elec;

// Electrons won't be recombined and thus don't need to know which box they are in

part[i].boxcol = NULL;

part[i].boxrow = NULL;

// Random spacing

if (random)  {

part[i].x=xctr + RandL()*spread;

part[i].y=yctr + RandL()*spread;

}

// Even spacing

else  {

// Original code to space particles uniformly in a 2-D region

//part[i].x=(i%nside+0.5)*delta;

//part[i].y=(i/nside+0.5)*delta;

// My code to space particles uniformly along a line

part[i].x = xctr;

//part[i].y = delta*(i+0.5);

part[i].y = yctr; // Assume for now we only have one particle to start. Change this when we start with more particles

}

}

// Output the particle locations to file

/*ofstream pt("partloc.txt");

pt << "i\tx\ty" << endl;

for (i=0; i<npart; i++)  {

pt << i << "\t" << part[i].x << "\t" << part[i].y << "\t"

<< part[i].u << "\t" << part[i].v << "\t" << endl;

}

pt.close();*/

return;

}

// Split the particle in half and place one delta apart

void splitparticle(int & npart, PARTICLE* & part, int partnum, double delta) {

addparticle(npart, part, -1, part[partnum].x+delta/2.0, part[partnum].y, part[partnum].weight/2.0, true, 0, 0);

part[partnum].weight = part[partnum].weight/2.0;

part[partnum].tot_charge = part[partnum].charge*part[partnum].weight*elec;

part[partnum].x = part[partnum].x-delta/2.0;

}

// Return the row that the particle is in

int getrow(double y, double dheight) {

return ((int)(NUMROW*y/dheight));

}

// Return the column that the particle is in

int getcol(double x, double dlength) {

return ((int)(NUMCOL*x/dlength));

}